Mt. Morgan JOB SAFETY AND ENVIRONMENTAL ANALYSIS

Daily Metallurgical Accounting and Plant Mass Balance

Job Safety and Environmental Analysis  ·  Heritage Minerals  ·  Mt. Morgan Processing Plant

| Document number | JSEA-PRO-MET-202-001 |
| --- | --- |
| Work instruction | None. This is a desk-based task with a JSEA line but no work instruction on the SWI-MET register. |
| Parent procedure | HM-PRC-VXX-PRO206 Metallurgical Accounting and Reconciliation |
| Area | 202 Metallurgical Accounting |
| Classification | Operating |
| Register line | JSEA-PRO-MET-202-001 in HM-MMM-WHS-REG-JSEA-PRO-updated_10, tab JSEA_MET |
| Process owner | Heritage Mineral Operations · Business unit Mt. Morgan |
| Document owner | Process Operations Superintendent |
| Document approver | Process Manager |
| Revision | Rev 0 |
| Date | 11 August 2026 |
| Prepared by | GGT (cost code 4034-033) |
| Reviewed by | ________________________ |
| Approved by | ________________________ |

# 1  PURPOSE / SCOPE

This job safety and environmental analysis covers the task daily metallurgical accounting and plant mass balance in 202  Metallurgical Accounting at the Mt. Morgan processing plant. It identifies the hazards at each step of the task, the controls required, and the residual risk once those controls are in place.

- Close the tonnes and grade balance across the plant for the day, from feed through to tailings and product.

- The daily balance is the first place a real loss shows up. If it is not closed every day, a loss can run for a month before anyone sees it.

It is read with HM-PRC-VXX-PRO206 Metallurgical Accounting and Reconciliation, the procedure this task sits under.

# 2  RESPONSIBILITIES

| Role | Responsibilities |
| --- | --- |
| Operator / Metallurgical Technician | Only carry out this task where competent and authorised to do so. Wear the PPE listed in Section 5 without exception. Stop the task and report immediately if any control in Section 4 is not in place. Record the work accurately. Never invent or back-fill a result that was not obtained. |
| Supervisor / Coordinator | Ensure the safety of all personnel carrying out this task. Understand that much of the plant is automated and equipment may start or stop without warning. Confirm the task is authorised and that the area is fit before it starts. Ensure this JSEA is communicated to everyone doing the task and that the sign-on sheet is completed. |
| Manager / Superintendent | Ensure all personnel have received adequate training and are competent. Ensure this JSEA is reviewed at the interval in Section 10 and after any incident or change. Approve the risk ranking in Section 4. |

# 3  ABBREVIATIONS / DEFINITIONS

| Term | Meaning |
| --- | --- |
| AVR | Acidification, volatilisation and regeneration - the cyanide recovery step in Area 081 |
| CIL | Carbon in leach |
| CN Free | Free cyanide, determined by silver nitrate titration |
| DO | Dissolved oxygen |
| HCN | Hydrogen cyanide gas |
| JSEA | Job safety and environmental analysis - this document |
| P80 | The screen size through which 80 per cent of the material passes |
| PPE | Personal protective equipment |
| ReCYN | The resin-in-pulp cyanide and metal recovery process used in Areas 041, 051, 071 and 081 |
| SWI | Standard work instruction - the step-by-step document that sits under this JSEA |
| SWP | Standard work procedure - the circuit-level procedure this task sits under |
| TSF | Tailings storage facility |
| WAD CN | Weak acid dissociable cyanide |

# 4  POTENTIAL HAZARDS

| Scale | Values |
| --- | --- |
| Probability | 1 Rare 2 Unlikely 3 Possible 4 Likely 5 Almost certain |
| Consequence | 1 Insignificant 2 Minor 3 Moderate 4 Major 5 Catastrophic |
| Risk rank | Probability x Consequence. 1-4 Low 5-9 Medium 10-14 High 15-25 Extreme. Scored after the controls in the same row are in place. |

| Job step | Hazards identified | Controls required | P | C | Risk | Actioned by |
| --- | --- | --- | --- | --- | --- | --- |
| 1. Collect the day's inputs - dry tonnes to the mill, feed grade, circuit inventories, product weights and assays, tailings tonnes and grade. | Non-representative sample leading to a wrong process decision | Line flushed for the stated time before collecting. Sample taken at the same point, in the same way and at the same frequency every time. Container labelled BEFORE the sample is taken, with point, date, time and shift. Steady-state confirmed for 15 to 20 minutes before sampling. A sample taken off-condition is marked as such or discarded. | 3 | 3 | 9 MEDIUM | Metallurgist |
| 2. Confirm every input is traceable to a signed source. An unsourced number does not go into the balance. | Non-representative sample leading to a wrong process decision | Line flushed for the stated time before collecting. Sample taken at the same point, in the same way and at the same frequency every time. Container labelled BEFORE the sample is taken, with point, date, time and shift. Steady-state confirmed for 15 to 20 minutes before sampling. A sample taken off-condition is marked as such or discarded. | 3 | 3 | 9 MEDIUM | Metallurgist |
| 3. Reconcile solution and solids around each circuit in turn, not just around the plant as a whole - a plant-level balance can close while two circuits are wrong in opposite directions. | Non-representative sample leading to a wrong process decision | Line flushed for the stated time before collecting. Sample taken at the same point, in the same way and at the same frequency every time. Container labelled BEFORE the sample is taken, with point, date, time and shift. Steady-state confirmed for 15 to 20 minutes before sampling. A sample taken off-condition is marked as such or discarded. | 3 | 3 | 9 MEDIUM | Metallurgist |
| 4. Calculate recovery by circuit and for the plant. | Non-representative sample leading to a wrong process decision | Line flushed for the stated time before collecting. Sample taken at the same point, in the same way and at the same frequency every time. Container labelled BEFORE the sample is taken, with point, date, time and shift. Steady-state confirmed for 15 to 20 minutes before sampling. A sample taken off-condition is marked as such or discarded. | 3 | 3 | 9 MEDIUM | Metallurgist |
| 5. Compare against the previous day and against the month to date. Investigate any step change before reporting it. | Non-representative sample leading to a wrong process decision | Line flushed for the stated time before collecting. Sample taken at the same point, in the same way and at the same frequency every time. Container labelled BEFORE the sample is taken, with point, date, time and shift. Steady-state confirmed for 15 to 20 minutes before sampling. A sample taken off-condition is marked as such or discarded. | 3 | 3 | 9 MEDIUM | Metallurgist |
| 6. Record the variance against the agreed tolerance. Never adjust an input to force the balance to close. | Non-representative sample leading to a wrong process decision | Line flushed for the stated time before collecting. Sample taken at the same point, in the same way and at the same frequency every time. Container labelled BEFORE the sample is taken, with point, date, time and shift. Steady-state confirmed for 15 to 20 minutes before sampling. A sample taken off-condition is marked as such or discarded. | 3 | 3 | 9 MEDIUM | Metallurgist |
| 7. Have the closed balance cross-checked by a second person before it is issued. | Non-representative sample leading to a wrong process decision | Line flushed for the stated time before collecting. Sample taken at the same point, in the same way and at the same frequency every time. Container labelled BEFORE the sample is taken, with point, date, time and shift. Steady-state confirmed for 15 to 20 minutes before sampling. A sample taken off-condition is marked as such or discarded. | 3 | 3 | 9 MEDIUM | Metallurgist |
| 8. Issue to the distribution list and file the working. | An accounting or reconciliation error that hides a real metal loss | Every input traceable to a signed source - weighbridge ticket, laboratory certificate, survey sheet. Inputs entered once and cross-checked by a second person before the balance is closed. Any variance outside the agreed tolerance investigated and recorded before the period is signed off, never adjusted to force a balance. | 3 | 4 | 12 HIGH | Metallurgist |

# 5  PPE

Standard PPE is worn for all normal operations in the Mt. Morgan gold processing plant. The following is required for this task.

- Hard hat

- Safety glasses

- High-visibility long-sleeved shirt and long trousers

- Safety boots

- Gloves appropriate to the task

# 6  PROCEDURE

6.1  Introduction

Warning: many operations on the gold processing plant are automated and equipment may start or stop without warning. It is the operator's responsibility to ensure the safety of all personnel working on the plant at all times.

Points and equipment covered by this task:

| Tag / point | Description |
| --- | --- |
| Whole plant | Feed, leach, CIL, adsorption, elution, gold room, copper circuit, tailings |

Frequency: Daily, closed before the morning production meeting.

6.2  Job steps

1.  Collect the day's inputs - dry tonnes to the mill, feed grade, circuit inventories, product weights and assays, tailings tonnes and grade.

2.  Confirm every input is traceable to a signed source. An unsourced number does not go into the balance.

3.  Reconcile solution and solids around each circuit in turn, not just around the plant as a whole - a plant-level balance can close while two circuits are wrong in opposite directions.

4.  Calculate recovery by circuit and for the plant.

5.  Compare against the previous day and against the month to date. Investigate any step change before reporting it.

6.  Record the variance against the agreed tolerance. Never adjust an input to force the balance to close.

7.  Have the closed balance cross-checked by a second person before it is issued.

8.  Issue to the distribution list and file the working.

Equipment required:

- Metallurgical accounting spreadsheet or system

- Weightometer and flow totaliser readings

- Laboratory assay certificates for the period

- Inventory readings from the previous close

# 7  FLOWCHART

Not applicable. This is a desk-based task and the step sequence is in Section 6.2.

# 8  TRAINING

Every person carrying out this task has been through this JSEA at a communication session recorded on the sign-on sheet at the end of this document.

- Site induction, current.

- Cyanide awareness and gas detection competency where the area requires it.

- Working at height competency where the task requires it.

- Competency in the determinations and calculations carried out in this task.

# 9  REFERENCE

- No direct source held. Written from first principles against the plant configuration.

- MM-4034-033-P-09 Tag Register (942 tags) and the mechanical equipment list

- 4034-PR-PRO-002 Sampling Protocol - the sample points the balance is built on

# 10  REVIEW

This JSEA is reviewed every 12 months, and immediately after any incident, near miss, plant change or change to the schedule affecting this task.

# JSEA COMMUNICATION AND AWARENESS

| Presented by | ________________________ |
| --- | --- |
| Date | ________________________ |
| JSEA reference | JSEA-PRO-MET-202-001 |

| Name | Position | Department | Signature |
| --- | --- | --- | --- |