Mt. Morgan JOB SAFETY AND ENVIRONMENTAL ANALYSIS

Gold and Silver Circuit Inventory and Lock-Up Reconciliation

Job Safety and Environmental Analysis  ·  Heritage Minerals  ·  Mt. Morgan Processing Plant

| Document number | JSEA-PRO-MET-202-004 |
| --- | --- |
| Work instruction | None. This is a desk-based task with a JSEA line but no work instruction on the SWI-MET register. |
| Parent procedure | HM-PRC-VXX-PRO206 Metallurgical Accounting and Reconciliation |
| Area | 202 Metallurgical Accounting |
| Classification | Safety-critical |
| Register line | JSEA-PRO-MET-202-004 in HM-MMM-WHS-REG-JSEA-PRO-updated_10, tab JSEA_MET |
| Process owner | Heritage Mineral Operations · Business unit Mt. Morgan |
| Document owner | Process Operations Superintendent |
| Document approver | Process Manager |
| Revision | Rev 0 |
| Date | 11 August 2026 |
| Prepared by | GGT (cost code 4034-033) |
| Reviewed by | ________________________ |
| Approved by | ________________________ |

# 1  PURPOSE / SCOPE

This job safety and environmental analysis covers the task gold and silver circuit inventory and lock-up reconciliation in 202  Metallurgical Accounting at the Mt. Morgan processing plant. It identifies the hazards at each step of the task, the controls required, and the residual risk once those controls are in place.

- Count what gold and silver is actually held in circuit at the close - on carbon, on resin, in solution, in the elution circuit, in the gold room and in lock-up.

- Lock-up is the largest single uncertainty in the metal balance and the one most exposed to a security incident.

It is read with HM-PRC-VXX-PRO206 Metallurgical Accounting and Reconciliation, the procedure this task sits under.

# 2  RESPONSIBILITIES

| Role | Responsibilities |
| --- | --- |
| Operator / Metallurgical Technician | Only carry out this task where competent and authorised to do so. Wear the PPE listed in Section 5 without exception. Stop the task and report immediately if any control in Section 4 is not in place. Record the work accurately. Never invent or back-fill a result that was not obtained. |
| Supervisor / Coordinator | Ensure the safety of all personnel carrying out this task. Understand that much of the plant is automated and equipment may start or stop without warning. Confirm the task is authorised and that the area is fit before it starts. Ensure this JSEA is communicated to everyone doing the task and that the sign-on sheet is completed. |
| Manager / Superintendent | Ensure all personnel have received adequate training and are competent. Ensure this JSEA is reviewed at the interval in Section 10 and after any incident or change. Approve the risk ranking in Section 4. |

# 3  ABBREVIATIONS / DEFINITIONS

| Term | Meaning |
| --- | --- |
| AVR | Acidification, volatilisation and regeneration - the cyanide recovery step in Area 081 |
| CIL | Carbon in leach |
| CN Free | Free cyanide, determined by silver nitrate titration |
| DO | Dissolved oxygen |
| HCN | Hydrogen cyanide gas |
| JSEA | Job safety and environmental analysis - this document |
| P80 | The screen size through which 80 per cent of the material passes |
| PPE | Personal protective equipment |
| ReCYN | The resin-in-pulp cyanide and metal recovery process used in Areas 041, 051, 071 and 081 |
| SWI | Standard work instruction - the step-by-step document that sits under this JSEA |
| SWP | Standard work procedure - the circuit-level procedure this task sits under |
| TSF | Tailings storage facility |
| WAD CN | Weak acid dissociable cyanide |

# 4  POTENTIAL HAZARDS

| Scale | Values |
| --- | --- |
| Probability | 1 Rare 2 Unlikely 3 Possible 4 Likely 5 Almost certain |
| Consequence | 1 Insignificant 2 Minor 3 Moderate 4 Major 5 Catastrophic |
| Risk rank | Probability x Consequence. 1-4 Low 5-9 Medium 10-14 High 15-25 Extreme. Scored after the controls in the same row are in place. |

| Job step | Hazards identified | Controls required | P | C | Risk | Actioned by |
| --- | --- | --- | --- | --- | --- | --- |
| 1. Arrange the count with the Shift Supervisor so the circuit is at a known, stable state. | Exposure to a gold security incident, and to suspicion in one | Two-person rule in the gold room, lock-up and bullion areas. Access controlled and logged. Nothing carried in or out except through the controlled route. Every movement recorded and countersigned. Never work alone with unsecured product. | 2 | 4 | 8 MEDIUM | Metallurgist |
| 2. Take carbon and resin concentrations per tank from the sampling instructions, and multiply by the confirmed working volume of each vessel - not the nameplate volume. | Hydrogen cyanide gas released at the open sample point Fall from a tank top, launder platform, column platform or access ladder | Personal HCN monitor worn and switched on, in calibration date. Fixed detection confirmed healthy - alarm 5 ppm, high-high 10 ppm. Sample point approached from upwind. pH of the stream confirmed above 10.5 before opening. Second person in sight and clear. Cyanide antidote kit on site and a trained first aider on shift. Withdraw upwind and report on any reading above the site action level. Fixed platforms, stairways and handrails used in preference to any other access. Harness and fall arrest where no fixed platform exists. Three points of contact on ladders. No sampling at height alone. Sample containers carried in a bag or hoisted, not in the hand on a ladder. | 3 | 5 | 15 EXTREME | Metallurgist |
| 3. Take loading assays for carbon and resin from the laboratory for the same date. | Exposure to a gold security incident, and to suspicion in one | Two-person rule in the gold room, lock-up and bullion areas. Access controlled and logged. Nothing carried in or out except through the controlled route. Every movement recorded and countersigned. Never work alone with unsecured product. | 2 | 4 | 8 MEDIUM | Metallurgist |
| 4. Count the gold room lock-up under the two-person rule. Every item weighed, recorded and countersigned by both. | Mercury vapour in the gold room, elution and retort areas | Area ventilation confirmed running before entry. Mercury vapour monitoring where the site procedure requires it. No eating, drinking or smoking. Hands washed before leaving the area. | 2 | 4 | 8 MEDIUM | Metallurgist |
| 5. Account for material in transit - carbon in an elution batch, resin in a transfer, sludge not yet smelted. | Mercury vapour in the gold room, elution and retort areas | Area ventilation confirmed running before entry. Mercury vapour monitoring where the site procedure requires it. No eating, drinking or smoking. Hands washed before leaving the area. | 2 | 4 | 8 MEDIUM | Metallurgist |
| 6. Reconcile the count against the previous close plus movements. Investigate any difference before signing. | Exposure to a gold security incident, and to suspicion in one | Two-person rule in the gold room, lock-up and bullion areas. Access controlled and logged. Nothing carried in or out except through the controlled route. Every movement recorded and countersigned. Never work alone with unsecured product. | 2 | 4 | 8 MEDIUM | Metallurgist |
| 7. Have both counters sign the count sheet, and the Metallurgist and Process Manager approve it. | Exposure to a gold security incident, and to suspicion in one | Two-person rule in the gold room, lock-up and bullion areas. Access controlled and logged. Nothing carried in or out except through the controlled route. Every movement recorded and countersigned. Never work alone with unsecured product. | 2 | 4 | 8 MEDIUM | Metallurgist |

# 5  PPE

Standard PPE is worn for all normal operations in the Mt. Morgan gold processing plant. The following is required for this task.

- Hard hat

- Safety glasses

- High-visibility long-sleeved shirt and long trousers

- Safety boots

- Gloves appropriate to the task

- Personal HCN gas monitor, calibrated and in test date

- Chemical splash goggles and face shield

- Nitrile rubber gloves

- Nitrile PVC boots

- Chemical resistant suit

- Full or half gas mask with A2B2E2K2P3 cartridge, carried and available for immediate use

- Harness and fall arrest where no fixed platform exists

# 6  PROCEDURE

6.1  Introduction

Warning: many operations on the gold processing plant are automated and equipment may start or stop without warning. It is the operator's responsibility to ensure the safety of all personnel working on the plant at all times.

Points and equipment covered by this task:

| Tag / point | Description |
| --- | --- |
| 032 | Carbon inventory in the CIL train |
| 041 / 051 | Resin inventory in the adsorption trains |
| 061 | Elution circuit solution and carbon in transit |
| 062 | Gold room - sludge, cathodes, doré, lock-up |

Frequency: Monthly at the close, and after any circuit shutdown or major upset.

6.2  Job steps

1.  Arrange the count with the Shift Supervisor so the circuit is at a known, stable state.

2.  Take carbon and resin concentrations per tank from the sampling instructions, and multiply by the confirmed working volume of each vessel - not the nameplate volume.

3.  Take loading assays for carbon and resin from the laboratory for the same date.

4.  Count the gold room lock-up under the two-person rule. Every item weighed, recorded and countersigned by both.

5.  Account for material in transit - carbon in an elution batch, resin in a transfer, sludge not yet smelted.

6.  Reconcile the count against the previous close plus movements. Investigate any difference before signing.

7.  Have both counters sign the count sheet, and the Metallurgist and Process Manager approve it.

Equipment required:

- Carbon and resin concentration results from the sampling round

- Circuit volumes from the equipment list

- Gold room lock-up register

- Two-person count sheets

- Scales, calibrated and in date

# 7  FLOWCHART

Not applicable. This is a desk-based task and the step sequence is in Section 6.2.

# 8  TRAINING

Every person carrying out this task has been through this JSEA at a communication session recorded on the sign-on sheet at the end of this document.

- Site induction, current.

- Cyanide awareness and gas detection competency where the area requires it.

- Working at height competency where the task requires it.

- Competency in the determinations and calculations carried out in this task.

# 9  REFERENCE

- No direct source held. Written from first principles.

- Mechanical equipment list - vessel volumes

- MM-4034-033-RES Resin Line Schedule (268 lines)

- KBK-MIR-MP-PRO-MET-SOP-0044 Pengiriman Bullion, received 11 Aug 2026

# 10  REVIEW

This JSEA is reviewed every 12 months, and immediately after any incident, near miss, plant change or change to the schedule affecting this task.

# JSEA COMMUNICATION AND AWARENESS

| Presented by | ________________________ |
| --- | --- |
| Date | ________________________ |
| JSEA reference | JSEA-PRO-MET-202-004 |

| Name | Position | Department | Signature |
| --- | --- | --- | --- |