Mt. Morgan JOB SAFETY AND ENVIRONMENTAL ANALYSIS

Gold Elution, Electrowinning and Pre-Soak Solution Sampling

Job Safety and Environmental Analysis  ·  Heritage Minerals  ·  Mt. Morgan Processing Plant

| Document number | JSEA-PRO-MET-201-013 |
| --- | --- |
| Work instruction | SWI-PRO-MET-201-013 Gold Elution, Electrowinning and Pre-Soak Solution Sampling |
| Parent procedure | HM-PRC-VXX-PRO205 Plant Sampling and Metallurgical Data Collection |
| Area | 061 / 062 Gold recovery |
| Classification | Safety-critical |
| Register line | JSEA-PRO-MET-201-013 in HM-MMM-WHS-REG-JSEA-PRO-updated_10, tab JSEA_MET |
| Process owner | Heritage Mineral Operations · Business unit Mt. Morgan |
| Document owner | Process Operations Superintendent |
| Document approver | Process Manager |
| Revision | Rev 0 |
| Date | 11 August 2026 |
| Prepared by | GGT (cost code 4034-033) |
| Reviewed by | ________________________ |
| Approved by | ________________________ |

# 1  PURPOSE / SCOPE

This job safety and environmental analysis covers the task gold elution, electrowinning and pre-soak solution sampling in 061 / 062 Gold recovery at the Mt. Morgan processing plant. It identifies the hazards at each step of the task, the controls required, and the residual risk once those controls are in place.

- Track the gold through the elution batch - pre-soak, eluate, electrowinning feed and barren electrolyte.

- The eluate-to-barren-electrolyte pair sets electrowinning efficiency and gold in circuit.

It is read with work instruction SWI-PRO-MET-201-013, which gives the step-by-step method, and with HM-PRC-VXX-PRO205 Plant Sampling and Metallurgical Data Collection, the procedure this task sits under.

# 2  RESPONSIBILITIES

| Role | Responsibilities |
| --- | --- |
| Operator / Metallurgical Technician | Only carry out this task where competent and authorised to do so. Wear the PPE listed in Section 5 without exception. Stop the task and report immediately if any control in Section 4 is not in place. Record the work accurately. Never invent or back-fill a result that was not obtained. |
| Supervisor / Coordinator | Ensure the safety of all personnel carrying out this task. Understand that much of the plant is automated and equipment may start or stop without warning. Confirm the task is authorised and that the area is fit before it starts. Ensure this JSEA is communicated to everyone doing the task and that the sign-on sheet is completed. |
| Manager / Superintendent | Ensure all personnel have received adequate training and are competent. Ensure this JSEA is reviewed at the interval in Section 10 and after any incident or change. Approve the risk ranking in Section 4. |

# 3  ABBREVIATIONS / DEFINITIONS

| Term | Meaning |
| --- | --- |
| AVR | Acidification, volatilisation and regeneration - the cyanide recovery step in Area 081 |
| CIL | Carbon in leach |
| CN Free | Free cyanide, determined by silver nitrate titration |
| DO | Dissolved oxygen |
| HCN | Hydrogen cyanide gas |
| JSEA | Job safety and environmental analysis - this document |
| P80 | The screen size through which 80 per cent of the material passes |
| PPE | Personal protective equipment |
| ReCYN | The resin-in-pulp cyanide and metal recovery process used in Areas 041, 051, 071 and 081 |
| SWI | Standard work instruction - the step-by-step document that sits under this JSEA |
| SWP | Standard work procedure - the circuit-level procedure this task sits under |
| TSF | Tailings storage facility |
| WAD CN | Weak acid dissociable cyanide |

# 4  POTENTIAL HAZARDS

| Scale | Values |
| --- | --- |
| Probability | 1 Rare 2 Unlikely 3 Possible 4 Likely 5 Almost certain |
| Consequence | 1 Insignificant 2 Minor 3 Moderate 4 Major 5 Catastrophic |
| Risk rank | Probability x Consequence. 1-4 Low 5-9 Medium 10-14 High 15-25 Extreme. Scored after the controls in the same row are in place. |

| Job step | Hazards identified | Controls required | P | C | Risk | Actioned by |
| --- | --- | --- | --- | --- | --- | --- |
| 1. Confirm with the control room where the batch is in the elution sequence before approaching. | Contact with hot solution or hot surface at the elution, stripping or regeneration circuits Mercury vapour in the gold room, elution and retort areas Working alone in a cyanide area where a collapse would not be seen | Confirm the circuit is not on a heating cycle before sampling. Heat-resistant gloves and face shield. Allow the sample line to flush and cool before collecting. Do not sample a column under pressure. Area ventilation confirmed running before entry. Mercury vapour monitoring where the site procedure requires it. No eating, drinking or smoking. Hands washed before leaving the area. Cyanide-area sampling is not done alone. Control room notified at the start and end of every round. Radio check-in at the interval set by the Shift Supervisor. | 3 | 4 | 12 HIGH | Operator |
| 2. Do NOT sample a column that is under pressure or on a heating cycle. Confirm the circuit is at the sampling condition stated in the elution procedure. | Hydrogen cyanide gas released at the open sample point Contact with hot solution or hot surface at the elution, stripping or regeneration circuits Release from a pressurised sample line, hose or fitting | Personal HCN monitor worn and switched on, in calibration date. Fixed detection confirmed healthy - alarm 5 ppm, high-high 10 ppm. Sample point approached from upwind. pH of the stream confirmed above 10.5 before opening. Second person in sight and clear. Cyanide antidote kit on site and a trained first aider on shift. Withdraw upwind and report on any reading above the site action level. Confirm the circuit is not on a heating cycle before sampling. Heat-resistant gloves and face shield. Allow the sample line to flush and cool before collecting. Do not sample a column under pressure. Confirm the sample line is not blocked before opening. Open slowly and stand clear of the discharge path. Report any weeping fitting rather than tightening it under pressure. | 3 | 5 | 15 EXTREME | Operator |
| 3. Flush the sample line and let the first flow discharge to drain until the temperature at the point is steady. | Hydrogen cyanide gas released at the open sample point Contact with hot solution or hot surface at the elution, stripping or regeneration circuits Release from a pressurised sample line, hose or fitting | Personal HCN monitor worn and switched on, in calibration date. Fixed detection confirmed healthy - alarm 5 ppm, high-high 10 ppm. Sample point approached from upwind. pH of the stream confirmed above 10.5 before opening. Second person in sight and clear. Cyanide antidote kit on site and a trained first aider on shift. Withdraw upwind and report on any reading above the site action level. Confirm the circuit is not on a heating cycle before sampling. Heat-resistant gloves and face shield. Allow the sample line to flush and cool before collecting. Do not sample a column under pressure. Confirm the sample line is not blocked before opening. Open slowly and stand clear of the discharge path. Report any weeping fitting rather than tightening it under pressure. | 3 | 5 | 15 EXTREME | Operator |
| 4. Collect into a heat-resistant bottle held below the point, not in line with the discharge. | Hydrogen cyanide gas released at the open sample point Splash of cyanide-bearing slurry or solution to skin or eyes Contact with hot solution or hot surface at the elution, stripping or regeneration circuits | Personal HCN monitor worn and switched on, in calibration date. Fixed detection confirmed healthy - alarm 5 ppm, high-high 10 ppm. Sample point approached from upwind. pH of the stream confirmed above 10.5 before opening. Second person in sight and clear. Cyanide antidote kit on site and a trained first aider on shift. Withdraw upwind and report on any reading above the site action level. Sample valve opened slowly with the container positioned first and the body out of the spray path. Face shield, chemical splash goggles, chemical-resistant gloves and apron. Never open a sample valve whose discharge cannot be seen. Safety shower and eyewash proven flowing before the round starts. Confirm the circuit is not on a heating cycle before sampling. Heat-resistant gloves and face shield. Allow the sample line to flush and cool before collecting. Do not sample a column under pressure. | 3 | 5 | 15 EXTREME | Operator |
| 5. Allow the sample to cool before taking pH or titrating - a hot cyanide solution releases HCN faster and gives a false reading. | Hydrogen cyanide gas released at the open sample point Splash of cyanide-bearing slurry or solution to skin or eyes Contact with hot solution or hot surface at the elution, stripping or regeneration circuits | Personal HCN monitor worn and switched on, in calibration date. Fixed detection confirmed healthy - alarm 5 ppm, high-high 10 ppm. Sample point approached from upwind. pH of the stream confirmed above 10.5 before opening. Second person in sight and clear. Cyanide antidote kit on site and a trained first aider on shift. Withdraw upwind and report on any reading above the site action level. Sample valve opened slowly with the container positioned first and the body out of the spray path. Face shield, chemical splash goggles, chemical-resistant gloves and apron. Never open a sample valve whose discharge cannot be seen. Safety shower and eyewash proven flowing before the round starts. Confirm the circuit is not on a heating cycle before sampling. Heat-resistant gloves and face shield. Allow the sample line to flush and cool before collecting. Do not sample a column under pressure. | 3 | 5 | 15 EXTREME | Operator |
| 6. Bottle and label for gold, silver and copper assay. | Mercury vapour in the gold room, elution and retort areas Non-representative sample leading to a wrong process decision Cross-contamination from a container or cutter not cleaned between points | Area ventilation confirmed running before entry. Mercury vapour monitoring where the site procedure requires it. No eating, drinking or smoking. Hands washed before leaving the area. Line flushed for the stated time before collecting. Sample taken at the same point, in the same way and at the same frequency every time. Container labelled BEFORE the sample is taken, with point, date, time and shift. Steady-state confirmed for 15 to 20 minutes before sampling. A sample taken off-condition is marked as such or discarded. Bucket, container, cutter and sieve rinsed with process water and then with the stream being sampled, and the rinse discarded, before every sample. Dedicated containers for high-grade streams. Labels checked against the schedule before handover. | 3 | 4 | 12 HIGH | Operator |
| 7. Record batch number, elution stage, solution temperature and flow at the time of sampling. | Hydrogen cyanide gas released at the open sample point Contact with hot solution or hot surface at the elution, stripping or regeneration circuits Mercury vapour in the gold room, elution and retort areas | Personal HCN monitor worn and switched on, in calibration date. Fixed detection confirmed healthy - alarm 5 ppm, high-high 10 ppm. Sample point approached from upwind. pH of the stream confirmed above 10.5 before opening. Second person in sight and clear. Cyanide antidote kit on site and a trained first aider on shift. Withdraw upwind and report on any reading above the site action level. Confirm the circuit is not on a heating cycle before sampling. Heat-resistant gloves and face shield. Allow the sample line to flush and cool before collecting. Do not sample a column under pressure. Area ventilation confirmed running before entry. Mercury vapour monitoring where the site procedure requires it. No eating, drinking or smoking. Hands washed before leaving the area. | 3 | 5 | 15 EXTREME | Operator |

# 5  PPE

Standard PPE is worn for all normal operations in the Mt. Morgan gold processing plant. The following is required for this task.

- Hard hat

- Safety glasses

- High-visibility long-sleeved shirt and long trousers

- Safety boots

- Gloves appropriate to the task

- Personal HCN gas monitor, calibrated and in test date

- Chemical splash goggles and face shield

- Nitrile rubber gloves

- Nitrile PVC boots

- Chemical resistant suit

- Full or half gas mask with A2B2E2K2P3 cartridge, carried and available for immediate use

- Heat-resistant gloves

- Face shield

- Harness and fall arrest where no fixed platform exists

# 6  PROCEDURE

6.1  Introduction

Warning: many operations on the gold processing plant are automated and equipment may start or stop without warning. It is the operator's responsibility to ensure the safety of all personnel working on the plant at all times.

Points and equipment covered by this task:

| Tag / point | Description |
| --- | --- |
| 061-TK-060 | Pre-soak / eluate tank |
| 061-CL-010 | Acid wash column |
| 061-HX-001 / 061-HX-002 | Elution heat exchangers - solution temperature read at sampling |
| 062 | Electrowinning cells - feed and barren electrolyte |

Frequency: Each elution batch - pre-soak, mid-strip, end-of-strip, and electrowinning feed and barren each shift the cells are running.

6.2  Job steps

1.  Confirm with the control room where the batch is in the elution sequence before approaching.

2.  Do NOT sample a column that is under pressure or on a heating cycle. Confirm the circuit is at the sampling condition stated in the elution procedure.

3.  Flush the sample line and let the first flow discharge to drain until the temperature at the point is steady.

4.  Collect into a heat-resistant bottle held below the point, not in line with the discharge.

5.  Allow the sample to cool before taking pH or titrating - a hot cyanide solution releases HCN faster and gives a false reading.

6.  Bottle and label for gold, silver and copper assay.

7.  Record batch number, elution stage, solution temperature and flow at the time of sampling.

Equipment required:

- Sample bottles 250 mL, heat-resistant

- pH meter

- Thermometer or infrared thermometer

- Free cyanide titration kit and rhodanine indicator

- Filter paper and funnel

- Personal protective equipment as Section 5, including heat-resistant gloves

# 7  FLOWCHART

Not reproduced in this document. The step sequence is in work instruction SWI-PRO-MET-201-013 Section 6, which is the controlled version of the method.

# 8  TRAINING

Every person carrying out this task has been through this JSEA at a communication session recorded on the sign-on sheet at the end of this document, and is trained and signed off against work instruction SWI-PRO-MET-201-013.

- Site induction, current.

- Cyanide awareness and gas detection competency where the area requires it.

- Working at height competency where the task requires it.

- Competency in the determinations and calculations carried out in this task.

# 9  REFERENCE

- 4034-PR-PRO-002 Sampling Protocol, Gold Recovery table

- 4034-013-VE-028 Rev A Applied Heat Thermomat 2300 O&M manual (061-BR-001, 061-HE-001, 061-HX-001/002, 061-PP-022, 061-TK-060), received 11 Aug 2026

- HM-PRC-V01-PRO010 Gold Elution Pre-Soak Mixing, PRO011 Gold Elution, PRO013 Gold Electrowinning

# 10  REVIEW

This JSEA is reviewed every 12 months, and immediately after any incident, near miss, plant change or change to the schedule affecting this task.

# JSEA COMMUNICATION AND AWARENESS

| Presented by | ________________________ |
| --- | --- |
| Date | ________________________ |
| JSEA reference | JSEA-PRO-MET-201-013 |

| Name | Position | Department | Signature |
| --- | --- | --- | --- |