Mt. Morgan JOB SAFETY AND ENVIRONMENTAL ANALYSIS

Reagent Solution Strength Sampling - NaCN, NaOH, H2SO4, NaCl, Lime and NaSH

Job Safety and Environmental Analysis  ·  Heritage Minerals  ·  Mt. Morgan Processing Plant

| Document number | JSEA-PRO-MET-201-020 |
| --- | --- |
| Work instruction | SWI-PRO-MET-201-020 Reagent Solution Strength Sampling - NaCN, NaOH, H2SO4, NaCl, Lime and NaSH |
| Parent procedure | HM-PRC-VXX-PRO205 Plant Sampling and Metallurgical Data Collection |
| Area | 101 to 107 Reagents |
| Classification | Safety-critical |
| Register line | JSEA-PRO-MET-201-020 in HM-MMM-WHS-REG-JSEA-PRO-updated_10, tab JSEA_MET |
| Process owner | Heritage Mineral Operations · Business unit Mt. Morgan |
| Document owner | Process Operations Superintendent |
| Document approver | Process Manager |
| Revision | Rev 0 |
| Date | 11 August 2026 |
| Prepared by | GGT (cost code 4034-033) |
| Reviewed by | ________________________ |
| Approved by | ________________________ |

# 1  PURPOSE / SCOPE

This job safety and environmental analysis covers the task reagent solution strength sampling - nacn, naoh, h2so4, nacl, lime and nash in 101 to 107 Reagents at the Mt. Morgan processing plant. It identifies the hazards at each step of the task, the controls required, and the residual risk once those controls are in place.

- Confirm every made-up reagent is at the strength the process assumes before it is dosed.

- A reagent made up off-strength is dosed at the wrong rate everywhere it is used, and the error is not visible until the circuit drifts.

It is read with work instruction SWI-PRO-MET-201-020, which gives the step-by-step method, and with HM-PRC-VXX-PRO205 Plant Sampling and Metallurgical Data Collection, the procedure this task sits under.

# 2  RESPONSIBILITIES

| Role | Responsibilities |
| --- | --- |
| Operator / Metallurgical Technician | Only carry out this task where competent and authorised to do so. Wear the PPE listed in Section 5 without exception. Stop the task and report immediately if any control in Section 4 is not in place. Record the work accurately. Never invent or back-fill a result that was not obtained. |
| Supervisor / Coordinator | Ensure the safety of all personnel carrying out this task. Understand that much of the plant is automated and equipment may start or stop without warning. Confirm the task is authorised and that the area is fit before it starts. Ensure this JSEA is communicated to everyone doing the task and that the sign-on sheet is completed. |
| Manager / Superintendent | Ensure all personnel have received adequate training and are competent. Ensure this JSEA is reviewed at the interval in Section 10 and after any incident or change. Approve the risk ranking in Section 4. |

# 3  ABBREVIATIONS / DEFINITIONS

| Term | Meaning |
| --- | --- |
| AVR | Acidification, volatilisation and regeneration - the cyanide recovery step in Area 081 |
| CIL | Carbon in leach |
| CN Free | Free cyanide, determined by silver nitrate titration |
| DO | Dissolved oxygen |
| HCN | Hydrogen cyanide gas |
| JSEA | Job safety and environmental analysis - this document |
| P80 | The screen size through which 80 per cent of the material passes |
| PPE | Personal protective equipment |
| ReCYN | The resin-in-pulp cyanide and metal recovery process used in Areas 041, 051, 071 and 081 |
| SWI | Standard work instruction - the step-by-step document that sits under this JSEA |
| SWP | Standard work procedure - the circuit-level procedure this task sits under |
| TSF | Tailings storage facility |
| WAD CN | Weak acid dissociable cyanide |

# 4  POTENTIAL HAZARDS

| Scale | Values |
| --- | --- |
| Probability | 1 Rare 2 Unlikely 3 Possible 4 Likely 5 Almost certain |
| Consequence | 1 Insignificant 2 Minor 3 Moderate 4 Major 5 Catastrophic |
| Risk rank | Probability x Consequence. 1-4 Low 5-9 Medium 10-14 High 15-25 Extreme. Scored after the controls in the same row are in place. |

| Job step | Hazards identified | Controls required | P | C | Risk | Actioned by |
| --- | --- | --- | --- | --- | --- | --- |
| 1. Read the reagent-specific site sampling procedure before starting - the PPE and the approach differ between cyanide, acid, caustic and NaSH. | Hydrogen cyanide gas released at the open sample point Contact with sulphuric or hydrochloric acid - burn to skin or eyes Contact with sodium hydroxide solution - burn to skin or eyes | Personal HCN monitor worn and switched on, in calibration date. Fixed detection confirmed healthy - alarm 5 ppm, high-high 10 ppm. Sample point approached from upwind. pH of the stream confirmed above 10.5 before opening. Second person in sight and clear. Cyanide antidote kit on site and a trained first aider on shift. Withdraw upwind and report on any reading above the site action level. Full acid PPE including face shield, acid-resistant gloves, apron and boots. Sample drawn only from the designated sample valve. Add acid to water, never the reverse. Safety shower and eyewash within reach and proven flowing. Spill kit at the point. Full chemical PPE including face shield and chemical-resistant gloves. Sample drawn slowly from the designated valve. Safety shower and eyewash within reach and proven flowing. | 3 | 5 | 15 EXTREME | Operator |
| 2. NEVER use a container that has held a different reagent. Dedicated, labelled containers only. | Manual handling injury from carrying sample buckets, containers and composite drums Non-representative sample leading to a wrong process decision Cross-contamination from a container or cutter not cleaned between points | Sample volume kept to the schedule - no over-filling. Two-person lift or a trolley for composite containers above the site manual handling limit. Route cleared before lifting. Line flushed for the stated time before collecting. Sample taken at the same point, in the same way and at the same frequency every time. Container labelled BEFORE the sample is taken, with point, date, time and shift. Steady-state confirmed for 15 to 20 minutes before sampling. A sample taken off-condition is marked as such or discarded. Bucket, container, cutter and sieve rinsed with process water and then with the stream being sampled, and the rinse discarded, before every sample. Dedicated containers for high-grade streams. Labels checked against the schedule before handover. | 3 | 3 | 9 MEDIUM | Operator |
| 3. For sodium cyanide solution, sample only from the designated valve with full cyanide PPE, including the gas mask with A2B2E2K2P3 cartridge available for immediate use. | Hydrogen cyanide gas released at the open sample point Splash of cyanide-bearing slurry or solution to skin or eyes Non-representative sample leading to a wrong process decision | Personal HCN monitor worn and switched on, in calibration date. Fixed detection confirmed healthy - alarm 5 ppm, high-high 10 ppm. Sample point approached from upwind. pH of the stream confirmed above 10.5 before opening. Second person in sight and clear. Cyanide antidote kit on site and a trained first aider on shift. Withdraw upwind and report on any reading above the site action level. Sample valve opened slowly with the container positioned first and the body out of the spray path. Face shield, chemical splash goggles, chemical-resistant gloves and apron. Never open a sample valve whose discharge cannot be seen. Safety shower and eyewash proven flowing before the round starts. Line flushed for the stated time before collecting. Sample taken at the same point, in the same way and at the same frequency every time. Container labelled BEFORE the sample is taken, with point, date, time and shift. Steady-state confirmed for 15 to 20 minutes before sampling. A sample taken off-condition is marked as such or discarded. | 3 | 5 | 15 EXTREME | Operator |
| 4. For sodium cyanide briquettes, sample the solid with the dust controls in place and do not open the sample in an unventilated space. | Hydrogen cyanide gas released at the open sample point Inhalation of or contact with solid reagent - cyanide briquettes, lime or salt Splash of cyanide-bearing slurry or solution to skin or eyes | Personal HCN monitor worn and switched on, in calibration date. Fixed detection confirmed healthy - alarm 5 ppm, high-high 10 ppm. Sample point approached from upwind. pH of the stream confirmed above 10.5 before opening. Second person in sight and clear. Cyanide antidote kit on site and a trained first aider on shift. Withdraw upwind and report on any reading above the site action level. Sampling of solid cyanide done only with the full cyanide PPE set including a gas mask with an A2B2E2K2P3 cartridge. Dust suppression and local extraction confirmed running. No dry sweeping. Sample valve opened slowly with the container positioned first and the body out of the spray path. Face shield, chemical splash goggles, chemical-resistant gloves and apron. Never open a sample valve whose discharge cannot be seen. Safety shower and eyewash proven flowing before the round starts. | 3 | 5 | 15 EXTREME | Operator |
| 5. For sulphuric acid and sodium hydroxide, sample slowly from the designated diluted-solution valve, never from the concentrated storage tank. | Hydrogen cyanide gas released at the open sample point Contact with sulphuric or hydrochloric acid - burn to skin or eyes Contact with sodium hydroxide solution - burn to skin or eyes | Personal HCN monitor worn and switched on, in calibration date. Fixed detection confirmed healthy - alarm 5 ppm, high-high 10 ppm. Sample point approached from upwind. pH of the stream confirmed above 10.5 before opening. Second person in sight and clear. Cyanide antidote kit on site and a trained first aider on shift. Withdraw upwind and report on any reading above the site action level. Full acid PPE including face shield, acid-resistant gloves, apron and boots. Sample drawn only from the designated sample valve. Add acid to water, never the reverse. Safety shower and eyewash within reach and proven flowing. Spill kit at the point. Full chemical PPE including face shield and chemical-resistant gloves. Sample drawn slowly from the designated valve. Safety shower and eyewash within reach and proven flowing. | 3 | 5 | 15 EXTREME | Operator |
| 6. For sodium hydrosulphide, confirm no acid dosing is running anywhere that could contact the stream, and wear the multi-gas monitor including H2S. | Contact with sulphuric or hydrochloric acid - burn to skin or eyes Hydrogen sulphide released from sodium hydrosulphide or from an acidified sulphide stream Working alone in a cyanide area where a collapse would not be seen | Full acid PPE including face shield, acid-resistant gloves, apron and boots. Sample drawn only from the designated sample valve. Add acid to water, never the reverse. Safety shower and eyewash within reach and proven flowing. Spill kit at the point. Personal multi-gas monitor including H2S worn and in calibration date. Area ventilation confirmed running. Two people, one clear. Withdraw upwind on any alarm. No sampling of the NaSH circuit during acid dosing. Cyanide-area sampling is not done alone. Control room notified at the start and end of every round. Radio check-in at the interval set by the Shift Supervisor. | 3 | 5 | 15 EXTREME | Operator |
| 7. Titrate or measure density in the field where the method allows, and bottle the laboratory split for confirmation. | Hydrogen cyanide gas released at the open sample point Contact with sulphuric or hydrochloric acid - burn to skin or eyes Contact with sodium hydroxide solution - burn to skin or eyes | Personal HCN monitor worn and switched on, in calibration date. Fixed detection confirmed healthy - alarm 5 ppm, high-high 10 ppm. Sample point approached from upwind. pH of the stream confirmed above 10.5 before opening. Second person in sight and clear. Cyanide antidote kit on site and a trained first aider on shift. Withdraw upwind and report on any reading above the site action level. Full acid PPE including face shield, acid-resistant gloves, apron and boots. Sample drawn only from the designated sample valve. Add acid to water, never the reverse. Safety shower and eyewash within reach and proven flowing. Spill kit at the point. Full chemical PPE including face shield and chemical-resistant gloves. Sample drawn slowly from the designated valve. Safety shower and eyewash within reach and proven flowing. | 3 | 5 | 15 EXTREME | Operator |
| 8. Record batch number, make-up time, measured strength and the target strength against each sample. Report any batch outside the target range to the Shift Supervisor before it is dosed. | Hydrogen cyanide gas released at the open sample point Splash of cyanide-bearing slurry or solution to skin or eyes Non-representative sample leading to a wrong process decision | Personal HCN monitor worn and switched on, in calibration date. Fixed detection confirmed healthy - alarm 5 ppm, high-high 10 ppm. Sample point approached from upwind. pH of the stream confirmed above 10.5 before opening. Second person in sight and clear. Cyanide antidote kit on site and a trained first aider on shift. Withdraw upwind and report on any reading above the site action level. Sample valve opened slowly with the container positioned first and the body out of the spray path. Face shield, chemical splash goggles, chemical-resistant gloves and apron. Never open a sample valve whose discharge cannot be seen. Safety shower and eyewash proven flowing before the round starts. Line flushed for the stated time before collecting. Sample taken at the same point, in the same way and at the same frequency every time. Container labelled BEFORE the sample is taken, with point, date, time and shift. Steady-state confirmed for 15 to 20 minutes before sampling. A sample taken off-condition is marked as such or discarded. | 3 | 5 | 15 EXTREME | Operator |

# 5  PPE

Standard PPE is worn for all normal operations in the Mt. Morgan gold processing plant. The following is required for this task.

- Hard hat

- Safety glasses

- High-visibility long-sleeved shirt and long trousers

- Safety boots

- Gloves appropriate to the task

- Personal HCN gas monitor, calibrated and in test date

- Chemical splash goggles and face shield

- Nitrile rubber gloves

- Nitrile PVC boots

- Chemical resistant suit

- Full or half gas mask with A2B2E2K2P3 cartridge, carried and available for immediate use

- Acid-resistant gloves, apron and face shield

- Chemical splash goggles

- Personal multi-gas monitor including H2S, calibrated and in test date

# 6  PROCEDURE

6.1  Introduction

Warning: many operations on the gold processing plant are automated and equipment may start or stop without warning. It is the operator's responsibility to ensure the safety of all personnel working on the plant at all times.

Points and equipment covered by this task:

| Tag / point | Description |
| --- | --- |
| 101 | Sodium cyanide - solution and briquettes |
| 102 | Sodium hydroxide - diluted |
| 103 | Lime - slaked slurry |
| 104 | Sodium chloride - made-up solution |
| 105 | Sulphuric acid - diluted |
| 107 | Sodium hydrosulphide - as delivered and dosed |

Frequency: Every batch made up, and once per shift on the standing solutions.

6.2  Job steps

1.  Read the reagent-specific site sampling procedure before starting - the PPE and the approach differ between cyanide, acid, caustic and NaSH.

2.  NEVER use a container that has held a different reagent. Dedicated, labelled containers only.

3.  For sodium cyanide solution, sample only from the designated valve with full cyanide PPE, including the gas mask with A2B2E2K2P3 cartridge available for immediate use.

4.  For sodium cyanide briquettes, sample the solid with the dust controls in place and do not open the sample in an unventilated space.

5.  For sulphuric acid and sodium hydroxide, sample slowly from the designated diluted-solution valve, never from the concentrated storage tank.

6.  For sodium hydrosulphide, confirm no acid dosing is running anywhere that could contact the stream, and wear the multi-gas monitor including H2S.

7.  Titrate or measure density in the field where the method allows, and bottle the laboratory split for confirmation.

8.  Record batch number, make-up time, measured strength and the target strength against each sample. Report any batch outside the target range to the Shift Supervisor before it is dosed.

Equipment required:

- Sample bottles 250 mL, one dedicated set per reagent

- pH meter

- Hydrometer or density meter

- Titration set - burette, pipette, conical flask, stand, wash bottle

- Standardised AgNO3 for cyanide; standardised HCl for caustic; standardised NaOH for acid

- Rhodanine and phenolphthalein indicators

- Distilled water

- Personal protective equipment as Section 5, matched to the reagent being sampled

# 7  FLOWCHART

Not reproduced in this document. The step sequence is in work instruction SWI-PRO-MET-201-020 Section 6, which is the controlled version of the method.

# 8  TRAINING

Every person carrying out this task has been through this JSEA at a communication session recorded on the sign-on sheet at the end of this document, and is trained and signed off against work instruction SWI-PRO-MET-201-020.

- Site induction, current.

- Cyanide awareness and gas detection competency where the area requires it.

- Working at height competency where the task requires it.

- Competency in the determinations and calculations carried out in this task.

# 9  REFERENCE

- Mt. Morgan site procedures 19. Sodium Cyanide Solution, 20. Sodium Cyanide Briquettes, 21. Diluted Sulphuric Acid and 22. Diluted Sodium Hydroxide Sampling [LIVE]

- KBK-MIR-MP-PRO-MET-SOP-0009 caustic soda titration, SOP-0010 cyanide titration, SOP-0025 and SOP-0039/0040 hydrochloric acid titration, SOP-0041/0042 sulphuric acid titration and SOP-0043 AgNO3 standard preparation - the last six received 11 Aug 2026

- KBK-MIR-MP-PRO-OPE-SOP-0076 Fasilitas Cyanide, received 11 Aug 2026

- 4034-046-VE-019 Rev A Transmin quicklime slaking plant functional description, received 11 Aug 2026

- HM-PRC-V01-PRO021 to PRO027

# 10  REVIEW

This JSEA is reviewed every 12 months, and immediately after any incident, near miss, plant change or change to the schedule affecting this task.

# JSEA COMMUNICATION AND AWARENESS

| Presented by | ________________________ |
| --- | --- |
| Date | ________________________ |
| JSEA reference | JSEA-PRO-MET-201-020 |

| Name | Position | Department | Signature |
| --- | --- | --- | --- |