| Company: | Heritage Minerals Pty Ltd |  |  |  |  |  |  |  |  |  |  |  | ABN: |  |  |  | 61 633 121 954 |  |  |  |  |  |  |  |  |  |  | Date Developed: |  |  |  |  | 11-08-2026 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operation: | Mount Morgan Gold Mine |  |  |  |  |  |  |  |  |  |  |  | Supervisor: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Review Period: |  |  |  |  | 12 Months |  |  |  |  |
| Work Activity: | Laboratory Equipment Receipt, Installation and Commissioning Handover (305 Laboratory Services) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | JSEA/SWMS No.: |  |  |  |  | HM-MMM-WHS-FRM-031-R03-JSEA-PRO-LAB-305-016 |  |  |  |  |
| PERSONAL PROTECTIVE EQUIPMENT (PPE): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Steel Cap Boots |  | Gloves |  | Hard Hat & Glasses |  | Hearing Protection |  |  |  | Hi-Vis |  |  |  |  |  | Dust Mask |  |  |  |  |  |  |  |  | Harness |  | Isolation |  |  |  |  | Hand Washing Hygiene |  |  | Double Eye Protection |  |  |
| RELEVANT LEGISLATION / CODE OF PRACTICE / STANDARDS: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Mining & Quarrying Safety and Health Act |  |  |  |  |  |  | Work Health and Safety Regulation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | COP Managing Risk in the Workplace |  |  |  |  |  |  |  |  | COP Managing the Risk of Falls |  |  |  |  |  |
| Mining & Quarrying Safety & Health Reg |  |  |  |  |  |  | Environmental Protection Act |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | COP Managing Risks of Plant |  |  |  |  |  |  |  |  | COP Risks of Hazardous Chemicals |  |  |  |  |  |
| Work Health and Safety Act |  |  |  |  |  |  | Environment Protection Regulation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | COP Construction Work |  |  |  |  |  |  |  |  | COP Hazardous Manual Tasks |  |  |  |  |  |
| HAZARDOUS CHEMICALS / DANGEROUS GOODS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Chemical(s) to be Used: |  |  |  |  |  |  |  |  |  |  |  | Risk Assessed: |  |  |  |  |  |  |  |  | SDS Available: |  |  |  |  | If “No”, Explain Why: |  |  |  |  |  |  |  | Hazardous/Dangerous: |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | No |  |  |  |  |  | Yes |  |  | No |  |  |  |  |  |  |  |  |  | Yes |  | No |  |
|  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | No |  |  |  |  |  | Yes |  |  | No |  |  |  |  |  |  |  |  |  | Yes |  | No |  |
|  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | No |  |  |  |  |  | Yes |  |  | No |  |  |  |  |  |  |  |  |  | Yes |  | No |  |
| HIGH RISK WORK: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Excavation / Trenching |  |  |  | Falling from Height |  |  |  |  |  |  | Rigging / Lifting |  |  |  |  |  |  |  |  |  |  | Powered Mobile Plant |  |  |  |  |  |  |  | Scaffolding |  |  |  |  |  |  |  |
| Pressurised Equipment |  |  |  | Asbestos |  |  |  |  |  |  | Demolition |  |  |  |  |  |  |  |  |  |  | Structural Alteration |  |  |  |  |  |  |  | Flammable Atmospheres |  |  |  |  |  |  |  |
| Explosives |  |  |  | Pre-Cast Concrete |  |  |  |  |  |  | Electrical |  |  |  |  |  |  |  |  |  |  | Confined Space |  |  |  |  |  |  |  | Chemical/Fuel/Refridge line |  |  |  |  |  |  |  |
| Extremes of Temperature |  |  |  | Traffic Corridors |  |  |  |  |  |  | Risk of Drowning |  |  |  |  |  |  |  |  |  |  | Driving Work |  |  |  |  |  |  |  | Telecommunication Tower |  |  |  |  |  |  |  |
| PERMITS REQUIRED |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type: |  |  |  |  | Permit Required: |  |  |  | Permit No: |  |  |  |  |  |  |  |  |  | Type: |  |  |  |  |  |  |  |  |  | Permit Required: |  |  |  |  | Permit No: |  |  |  |
| Excavation Permit |  |  |  |  | Yes |  |  | No |  |  |  |  |  |  |  |  |  |  | Working at Heights Permit |  |  |  |  |  |  |  |  |  | Yes |  |  | No |  |  |  |  |  |
| Confined Space Permit |  |  |  |  | Yes |  |  | No |  |  |  |  |  |  |  |  |  |  | Hot Work Permit |  |  |  |  |  |  |  |  |  | Yes |  |  | No |  |  |  |  |  |
| MOBILE PLANT REQUIRED FOR TASK |  |  |  |  |  |  |  |  |  |  |  |  |  | TRAINING REQUIRED FOR TASK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Plant: |  |  |  |  | License: |  |  |  |  |  |  |  |  | Qualification: |  |  |  |  |  |  |  |  |  |  |  |  | Yes: |  | No: |  | Comments: |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | Construction Industry Induction Card |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | Heritage Minerals Site Induction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | Other: |  |  |  |  |  | Competency in the determinations and calculations carried out in this task |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | Other: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | Other: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HERITAGE MINERALS STANDARD OPERATING PROCEDURES: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work Health and Safety SOPs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Environmental SOPs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-001-R02-Legal Obligations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-ENV-SOP-001-R02-Hazardous Substances |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-002-R02-Risk Management |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-ENV-SOP-002-R02-Soil Management |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-003-R02-Consultation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-ENV-SOP-003-R02-Fire Management |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-004-R02-Incident Reporting |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-ENV-SOP-004-R02-Cultural Heritage |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-005-R02-Emergency Response |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-ENV-SOP-005-R02-Dust & Air Management |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-006-R02-Audit & Inspection |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-ENV-SOP-006-R02-Waste Management |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-007-R02-Drugs & Alcohol |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-ENV-SOP-007-R02-Water Management |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-008-R02-Permit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-ENV-SOP-008-R02-Noise & Vibration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-009-R02-Induction & Training |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-ENV-SOP-009-R02-Flora Management |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-010-R02-Working Over Water |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-ENV-SOP-010-R02-Fauna Management |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-011-R02-Working in Isolation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-ENV-SOP-011-R02-Dredging Operations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-012-R02-COVID-19 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Quality SOPs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-013-R02-HAZOP |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-QAL-SOP-001-R02-Management Review |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-014-R02-Change Management |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-QAL-SOP-002-R02-Objectives & Targets |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-015-R02-Electrical |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-QAL-SOP-003-R02-Monitoring & Measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-016-R02-Cranes & Rigging |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-QAL-SOP-004-R02-Document Control |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-017-R02-Working at Heights |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-QAL-SOP-005-R02-Procurement |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-018-R02-Confined Space |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-QAL-SOP-006-R02-Record Management |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-019-R02-Isolation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-QAL-SOP-007-R02-Pre-Planning |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-020-R02-Asbestos |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-QAL-SOP-008-R02-Custom Focus |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-021-R02-Fatigue |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-QAL-SOP-009-R02-Design Management |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-022-R02-Plant & Equipment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-QAL-SOP-010-R02-non-conformance |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-023-R02-Excavation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-QAL-SOP-011-R02-Tendering |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-024-R02-Light Vehicle |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-QAL-SOP-012-R02-Supplier Engagement |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-025-R02-Security |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-QAL-SOP-013-R02-HR-Human Resources |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS -SOP-026-R02-Bullying & Harassment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-QAL-SOP-014-R02-HR-Disciplinary |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS -SOP-027-R02-Disciplinary |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-QAL-SOP-015-R02-HR-Bullying & Harassment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS -SOP-028-R02-Issue Resolution |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-QAL-SOP-016-R02-HR-Issue Resolution |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-QAL-SOP-017-R02-Stakeholder Consultation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-QAL-SOP-018-R02-Crisis Management |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | HERITAGE MINERALS 5X5 RISK MATRIX |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RISK RATING: | LOW (1-4) Broadly Acceptable-Manage by Routine SOPs |  |  |  | MODERATE (4-9) Tolerable-with Identified Controls Fully Implemented |  |  | HIGH (10-19) Undesirable-Additional Controls Required to Reduce Risk |  | EXTREME (20-25) Intolerable-Do Not Start Activity |  |
| STOP WORK IF UNSAFE |  |  | CONSEQUENCE |  |  |  |  |  |  |  |  |
|  |  |  | 1 = Insignificant |  |  | 2 = Minor | 3 = Moderate |  | 4 = Major |  | 5 = Catastrophic |
|  |  |  | People-No injuries requiring treatment. Environment-Minor localised environmental harm rectified within hours. No protected habitat or species affected. |  |  | People-First aid treatment. Environment-Minor transient environmental harm that requires days for recovery. No protected habitat or species affected. | People-Medical treatment required, Restricted Work Injury. Environment-Significant environmental harm that requires weeks for recovery. Environmental incident involving protected species/habitat. |  | People-Injuries resulting in; lost time, permanent disability, amputation, or surgery. Environment-Very serious long term environmental harm or contamination, years to recover. Protected species habitat damage. |  | People-Single or multiple fatalities. Environment-Severe environmental harm or contamination resulting in permanent environmental damage. Endangered species and habitat destroyed. |
| LIKELIHOOD |  |  |  |  |  |  |  |  |  |  |  |
| 5 = Almost Certain | Is Expected to Occur |  | MODERATE (5) |  |  | HIGH (10) | HIGH (15) |  | EXTREME (20) |  | EXTREME (25) |
| 4 = Likely | Will Probably Occur in Most Circumstances |  | MODERATE (4) |  |  | MODERATE (8) | HIGH (12) |  | HIGH (16) |  | EXTREME (20) |
| 3 = Possible | Should Occur at Some Time |  | LOW (3) |  |  | MODERATE (6) | MODERATE (9) |  | HIGH (12) |  | HIGH (15) |
| 2 = Unlikely | Could Occur at Some Time |  | LOW (2) |  |  | LOW (4) | MODERATE (6) |  | MODERATE (8) |  | HIGH (10) |
| 1 - Rare | May Occur Only Once in 5 Years |  | LOW (1) |  |  | LOW (2) | LOW (3) |  | MODERATE (4) |  | MODERATE (5) |
| L = | LOW |  |  | Work may proceed after being approved by your supervisor and HSEQ Superintendent, record and review regularly as any equipment / people / work process or SOPs change. |  |  |  |  |  |  |  |
| M = | MODERATE |  |  | Work may proceed after being approved by your supervisor and HSEQ Superintendent, but controls must be monitored and reviewed regularly. |  |  |  |  |  |  |  |
| H = | HIGH |  |  | Work may NOT start report to your supervisor, who will report to site management for further mitigation of risk i.e., increase supervision and inspection, use of permit system, SSE and HSEQ Superintendent must approve. |  |  |  |  |  |  |  |
| E = | EXTREME |  |  | WORK MUST NOT START, report to you supervisor, the risk must be re-assessed to an acceptable level. Further controls are to include Increase supervision, inspection, permit system must be used, risk assessment workshop, SSE and HSEQ Superintendent must approve. |  |  |  |  |  |  |  |

| Likelihood (L): | 1 = Rare, 2 = Unlikely, 3 = Possible, 4 = Likely, 5 = Almost Certain |  |  |  |  | Consequence (C): |  | 1 = Insignificant, 2 = Minor, 3 = Moderate, 4 = Major, 5 = Catastrophic |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Risk Rating: | LOW (1-4) Broadly Acceptable-Manage by Routine SOPs |  | MODERATE (4-9) Tolerable-with Identified Controls Fully Implemented |  |  |  | HIGH (10-19) Undesirable-Additional Controls Required to Reduce Risk |  |  |  |  |  | EXTREME (20-25) Intolerable-Do Not Start Activity |  |  |  |  |  |  |
| Task / Step No. (Job Steps) | Potential Hazards (What Can Go Wrong) | Initial Risk (L x C=R) |  |  | Control Measures |  |  |  | Residual Risk (L x C=R) |  |  | Hierarchy of Controls Applied |  |  |  |  |  |  | Responsible Workers |
|  |  | L | C | R |  |  |  |  | L | C | R | Elimination |  | Substitution | Isolation | Engineering | Admin | PPE |  |
| 1. Check every delivery against the quotation line by line. The supply is ex works, so shortages and transit damage are found here or not at all. | - Release from a pressurised sample line, hose or fitting | 2 | 4 | M8 | - Confirm the sample line is not blocked before opening - Open slowly and stand clear of the discharge path - Report any weeping fitting rather than tightening it under pressure |  |  |  | 1 | 4 | M4 |  |  |  |  |  | x |  | Laboratory Technician |
| 2. Confirm the services each item needs are in place before installation - power, water, drainage, compressed air, extraction. | - Contact with live electrical equipment at a sample pump, cutter or local control station - Release from a pressurised sample line, hose or fitting | 2 | 5 | H10 | - No opening of electrical enclosures - Faults reported, not investigated - Isolation, lock-out, tag-out and test-for-dead by an authorised person before any electrical work - Confirm the sample line is not blocked before opening - Open slowly and stand clear of the discharge path - Report any weeping fitting rather than tightening it under pressure |  |  |  | 1 | 5 | M5 |  |  |  | x |  | x |  | Laboratory Technician |
| 3. Note the items needing work outside the vendor scope: the dust collector pulse-system wiring, its isolator, the GPO and the compressed air connection. | - Manual handling injury from carrying sample buckets, containers and composite drums - Release from a pressurised sample line, hose or fitting - Non-representative sample leading to a wrong process decision | 3 | 4 | H12 | - Sample volume kept to the schedule - no over-filling - Two-person lift or a trolley for composite containers above the site manual handling limit - Route cleared before lifting - Confirm the sample line is not blocked before opening - Open slowly and stand clear of the discharge path - Report any weeping fitting rather than tightening it under pressure - Line flushed for the stated time before collecting - Sample taken at the same point, in the same way and at the same frequency every time - Container labelled BEFORE the sample is taken, with point, date, time and shift - Steady-state confirmed for 15 to 20 minutes before sampling - A sample taken off-condition is marked as such or discarded |  |  |  | 1 | 4 | M4 |  |  |  |  |  | x |  | Laboratory Technician |
| 4. Register the 600 L air receiver as a pressure vessel and arrange its statutory inspection before it goes into service. | - Release from a pressurised sample line, hose or fitting | 2 | 4 | M8 | - Confirm the sample line is not blocked before opening - Open slowly and stand clear of the discharge path - Report any weeping fitting rather than tightening it under pressure |  |  |  | 1 | 4 | M4 |  |  |  |  |  | x |  | Laboratory Technician |
| 5. Install, level and connect each item per its manual. | - Manual handling injury from carrying sample buckets, containers and composite drums | 3 | 2 | M6 | - Sample volume kept to the schedule - no over-filling - Two-person lift or a trolley for composite containers above the site manual handling limit - Route cleared before lifting |  |  |  | 1 | 2 | L2 |  |  |  |  |  | x |  | Laboratory Technician |
| 6. Calibrate or verify every instrument and balance before it is used for any result, and file the certificates. | - Contact with live electrical equipment at a sample pump, cutter or local control station - Non-representative sample leading to a wrong process decision | 3 | 5 | H15 | - No opening of electrical enclosures - Faults reported, not investigated - Isolation, lock-out, tag-out and test-for-dead by an authorised person before any electrical work - Line flushed for the stated time before collecting - Sample taken at the same point, in the same way and at the same frequency every time - Container labelled BEFORE the sample is taken, with point, date, time and shift - Steady-state confirmed for 15 to 20 minutes before sampling - A sample taken off-condition is marked as such or discarded |  |  |  | 1 | 5 | M5 |  |  |  | x |  | x |  | Laboratory Technician |
| 7. Run a commissioning check against a known standard on each instrument and record the result. | - Contact with live electrical equipment at a sample pump, cutter or local control station - Non-representative sample leading to a wrong process decision | 3 | 5 | H15 | - No opening of electrical enclosures - Faults reported, not investigated - Isolation, lock-out, tag-out and test-for-dead by an authorised person before any electrical work - Line flushed for the stated time before collecting - Sample taken at the same point, in the same way and at the same frequency every time - Container labelled BEFORE the sample is taken, with point, date, time and shift - Steady-state confirmed for 15 to 20 minutes before sampling - A sample taken off-condition is marked as such or discarded |  |  |  | 1 | 5 | M5 |  |  |  | x |  | x |  | Laboratory Technician |
| 8. Enter every item in the asset register with serial number, location, calibration interval and PM requirement. | - Manual handling injury from carrying sample buckets, containers and composite drums | 3 | 2 | M6 | - Sample volume kept to the schedule - no over-filling - Two-person lift or a trolley for composite containers above the site manual handling limit - Route cleared before lifting |  |  |  | 1 | 2 | L2 |  |  |  |  |  | x |  | Laboratory Technician |
| 9. Hand over item by item and record what is accepted and what remains outstanding. | - Non-representative sample leading to a wrong process decision | 3 | 3 | M9 | - Line flushed for the stated time before collecting - Sample taken at the same point, in the same way and at the same frequency every time - Container labelled BEFORE the sample is taken, with point, date, time and shift - Steady-state confirmed for 15 to 20 minutes before sampling - A sample taken off-condition is marked as such or discarded |  |  |  | 1 | 3 | L3 |  |  |  |  |  | x |  | Laboratory Technician |

| EMERGENCY RESPONSE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Nearest Emergency Assembly Point? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Nearest Fire Extinguishers? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Nearest First Aid Kit? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Method of Communicating an Emergency? |  |  |  |  |  |  | Phone No.: |  |  |  |  |  |  |  |  |  |  | UHF Channell: |  |  |  |  |  |  |  |  |
| ONGOING MANAGEMENT REVIEW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Review No: |  | 01 |  |  | 02 |  |  | 03 |  |  |  | 04 |  | 05 |  |  | 06 |  |  | 07 |  | 08 |  |  |  | 09 |
| Name: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Date: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Initial: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WORKERS CONSULTED ON DEVELOPMENT OF JSEA/SWMS: E.G. WORKERS, SAFETY REPRESENTATIVE, SUBJECT MATTER EXPERT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Name: |  |  |  | Position: |  |  |  |  | Name: |  |  |  |  | Position: |  |  |  |  | Name: |  |  |  | Position: |  |  |  |
| APPROVAL (Can be Approved by SSE or Supervisor or HSEQ Superintendent) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Name: |  |  |  | Position: |  |  |  |  | Date: |  |  |  |  | Name: |  |  |  |  | Position: |  |  |  | Date: |  |  |  |
| CHANGE MANAGEMENT PAGE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Date and Time: |  |  | Step #: |  |  | Description of Change: |  |  |  |  |  |  |  |  | New Control Measures: |  |  |  |  |  |  |  |  | Change Approval: |  |  |
| SIGN ON PAGE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Date: | Full Name: |  |  |  |  |  |  |  |  |  | Signature: |  | Date: |  |  | Full Name: |  |  |  |  |  |  |  |  | Signature: |  |