Mt. Morgan JOB SAFETY AND ENVIRONMENTAL ANALYSIS

Gold Security and Sample Integrity Control

Job Safety and Environmental Analysis  ·  Heritage Minerals  ·  Mt. Morgan Processing Plant

| Document number | JSEA-PRO-LAB-305-011 |
| --- | --- |
| Work instruction | SWI-PRO-LAB-305-011 Gold Security and Sample Integrity Control |
| Parent procedure | HM-PRC-VXX-PRO216 Laboratory Services, Safety and Quality |
| Area | 305 Laboratory Services |
| Classification | Safety-critical |
| Register line | JSEA-PRO-LAB-305-011 in HM-MMM-WHS-REG-JSEA-PRO-updated_10, tab JSEA_LAB |
| Process owner | Heritage Mineral Operations · Business unit Mt. Morgan |
| Document owner | Process Operations Superintendent |
| Document approver | Process Manager |
| Revision | Rev 0 |
| Date | 11 August 2026 |
| Prepared by | GGT (cost code 4034-033) |
| Reviewed by | ________________________ |
| Approved by | ________________________ |

# 1  PURPOSE / SCOPE

This job safety and environmental analysis covers the task gold security and sample integrity control in 305  Laboratory Services at the Mt. Morgan processing plant. It identifies the hazards at each step of the task, the controls required, and the residual risk once those controls are in place.

- Control gold security and sample integrity in the laboratory.

- The laboratory holds high-grade pulps, bullion samples and the results the whole operation is measured on. It protects both the gold and the people working with it.

It is read with work instruction SWI-PRO-LAB-305-011, which gives the step-by-step method, and with HM-PRC-VXX-PRO216 Laboratory Services, Safety and Quality, the procedure this task sits under.

# 2  RESPONSIBILITIES

| Role | Responsibilities |
| --- | --- |
| Operator / Metallurgical Technician | Only carry out this task where competent and authorised to do so. Wear the PPE listed in Section 5 without exception. Stop the task and report immediately if any control in Section 4 is not in place. Record the work accurately. Never invent or back-fill a result that was not obtained. |
| Supervisor / Coordinator | Ensure the safety of all personnel carrying out this task. Understand that much of the plant is automated and equipment may start or stop without warning. Confirm the task is authorised and that the area is fit before it starts. Ensure this JSEA is communicated to everyone doing the task and that the sign-on sheet is completed. |
| Manager / Superintendent | Ensure all personnel have received adequate training and are competent. Ensure this JSEA is reviewed at the interval in Section 10 and after any incident or change. Approve the risk ranking in Section 4. |

# 3  ABBREVIATIONS / DEFINITIONS

| Term | Meaning |
| --- | --- |
| AVR | Acidification, volatilisation and regeneration - the cyanide recovery step in Area 081 |
| CIL | Carbon in leach |
| CN Free | Free cyanide, determined by silver nitrate titration |
| DO | Dissolved oxygen |
| HCN | Hydrogen cyanide gas |
| JSEA | Job safety and environmental analysis - this document |
| P80 | The screen size through which 80 per cent of the material passes |
| PPE | Personal protective equipment |
| ReCYN | The resin-in-pulp cyanide and metal recovery process used in Areas 041, 051, 071 and 081 |
| SWI | Standard work instruction - the step-by-step document that sits under this JSEA |
| SWP | Standard work procedure - the circuit-level procedure this task sits under |
| TSF | Tailings storage facility |
| WAD CN | Weak acid dissociable cyanide |

# 4  POTENTIAL HAZARDS

| Scale | Values |
| --- | --- |
| Probability | 1 Rare 2 Unlikely 3 Possible 4 Likely 5 Almost certain |
| Consequence | 1 Insignificant 2 Minor 3 Moderate 4 Major 5 Catastrophic |
| Risk rank | Probability x Consequence. 1-4 Low 5-9 Medium 10-14 High 15-25 Extreme. Scored after the controls in the same row are in place. |

| Job step | Hazards identified | Controls required | P | C | Risk | Actioned by |
| --- | --- | --- | --- | --- | --- | --- |
| 1. Control access to the bullion and high-grade areas. Access is logged. | Exposure to a gold security incident, and to suspicion in one | Two-person rule in the gold room, lock-up and bullion areas. Access controlled and logged. Nothing carried in or out except through the controlled route. Every movement recorded and countersigned. Never work alone with unsecured product. | 2 | 4 | 8 MEDIUM | Laboratory Technician |
| 2. Apply the two-person rule in the bullion area. It protects the person as much as the gold. | Exposure to a gold security incident, and to suspicion in one | Two-person rule in the gold room, lock-up and bullion areas. Access controlled and logged. Nothing carried in or out except through the controlled route. Every movement recorded and countersigned. Never work alone with unsecured product. | 2 | 4 | 8 MEDIUM | Laboratory Technician |
| 3. Store bullion samples and high-grade pulps in the safe between operations. | Exposure to a gold security incident, and to suspicion in one | Two-person rule in the gold room, lock-up and bullion areas. Access controlled and logged. Nothing carried in or out except through the controlled route. Every movement recorded and countersigned. Never work alone with unsecured product. | 2 | 4 | 8 MEDIUM | Laboratory Technician |
| 4. Record every movement of bullion-related material in the custody register, countersigned. | Exposure to a gold security incident, and to suspicion in one | Two-person rule in the gold room, lock-up and bullion areas. Access controlled and logged. Nothing carried in or out except through the controlled route. Every movement recorded and countersigned. Never work alone with unsecured product. | 2 | 4 | 8 MEDIUM | Laboratory Technician |
| 5. Reconcile mass in against mass out for every bullion batch. | Exposure to a gold security incident, and to suspicion in one | Two-person rule in the gold room, lock-up and bullion areas. Access controlled and logged. Nothing carried in or out except through the controlled route. Every movement recorded and countersigned. Never work alone with unsecured product. | 2 | 4 | 8 MEDIUM | Laboratory Technician |
| 6. Nothing leaves the controlled area except by the controlled route. | Exposure to a gold security incident, and to suspicion in one | Two-person rule in the gold room, lock-up and bullion areas. Access controlled and logged. Nothing carried in or out except through the controlled route. Every movement recorded and countersigned. Never work alone with unsecured product. | 2 | 4 | 8 MEDIUM | Laboratory Technician |
| 7. Protect result integrity as well as material integrity - results are not discussed or released outside the agreed route before they are verified. | Exposure to a gold security incident, and to suspicion in one | Two-person rule in the gold room, lock-up and bullion areas. Access controlled and logged. Nothing carried in or out except through the controlled route. Every movement recorded and countersigned. Never work alone with unsecured product. | 2 | 4 | 8 MEDIUM | Laboratory Technician |
| 8. Report any discrepancy immediately, however small. A small unexplained discrepancy is how a large one starts. | Exposure to a gold security incident, and to suspicion in one | Two-person rule in the gold room, lock-up and bullion areas. Access controlled and logged. Nothing carried in or out except through the controlled route. Every movement recorded and countersigned. Never work alone with unsecured product. | 2 | 4 | 8 MEDIUM | Laboratory Technician |

# 5  PPE

Standard PPE is worn for all normal operations in the Mt. Morgan gold processing plant. The following is required for this task.

- Hard hat

- Safety glasses

- High-visibility long-sleeved shirt and long trousers

- Safety boots

- Gloves appropriate to the task

- Personal HCN gas monitor, calibrated and in test date

- Chemical splash goggles and face shield

- Nitrile rubber gloves

- Nitrile PVC boots

- Chemical resistant suit

- Full or half gas mask with A2B2E2K2P3 cartridge, carried and available for immediate use

- Acid-resistant gloves, apron and face shield

- Chemical splash goggles

# 6  PROCEDURE

6.1  Introduction

Warning: many operations on the gold processing plant are automated and equipment may start or stop without warning. It is the operator's responsibility to ensure the safety of all personnel working on the plant at all times.

Frequency: Continuous.

6.2  Job steps

1.  Control access to the bullion and high-grade areas. Access is logged.

2.  Apply the two-person rule in the bullion area. It protects the person as much as the gold.

3.  Store bullion samples and high-grade pulps in the safe between operations.

4.  Record every movement of bullion-related material in the custody register, countersigned.

5.  Reconcile mass in against mass out for every bullion batch.

6.  Nothing leaves the controlled area except by the controlled route.

7.  Protect result integrity as well as material integrity - results are not discussed or released outside the agreed route before they are verified.

8.  Report any discrepancy immediately, however small. A small unexplained discrepancy is how a large one starts.

Equipment required:

- Safe NV1

- Controlled area access

- Sample custody register

- Two-person rule

# 7  FLOWCHART

Not reproduced in this document. The step sequence is in work instruction SWI-PRO-LAB-305-011 Section 6, which is the controlled version of the method.

# 8  TRAINING

Every person carrying out this task has been through this JSEA at a communication session recorded on the sign-on sheet at the end of this document, and is trained and signed off against work instruction SWI-PRO-LAB-305-011.

- Site induction, current.

- Cyanide awareness and gas detection competency where the area requires it.

- Working at height competency where the task requires it.

- Competency in the determinations and calculations carried out in this task.

# 9  REFERENCE

- ILS quote Q0023979: Safe NV1 single shelf in the bullion assay area

- Heritage register line JSEA-PRO-LAB-305-011, tab JSEA_LAB / SWI-LAB

# 10  REVIEW

This JSEA is reviewed every 12 months, and immediately after any incident, near miss, plant change or change to the schedule affecting this task.

# JSEA COMMUNICATION AND AWARENESS

| Presented by | ________________________ |
| --- | --- |
| Date | ________________________ |
| JSEA reference | JSEA-PRO-LAB-305-011 |

| Name | Position | Department | Signature |
| --- | --- | --- | --- |