Mt. Morgan JOB SAFETY AND ENVIRONMENTAL ANALYSIS

Plant Survey Planning, Permitting and Execution

Job Safety and Environmental Analysis  ·  Heritage Minerals  ·  Mt. Morgan Processing Plant

| Document number | JSEA-PRO-MET-203-001 |
| --- | --- |
| Work instruction | SWI-PRO-MET-203-001 Plant Survey Planning, Permitting and Execution |
| Parent procedure | HM-PRC-VXX-PRO207 Plant Surveys and Circuit Performance Assessment |
| Area | 203 Plant Surveys |
| Classification | Safety-critical |
| Register line | JSEA-PRO-MET-203-001 in HM-MMM-WHS-REG-JSEA-PRO-updated_10, tab JSEA_MET |
| Process owner | Heritage Mineral Operations · Business unit Mt. Morgan |
| Document owner | Process Operations Superintendent |
| Document approver | Process Manager |
| Revision | Rev 0 |
| Date | 11 August 2026 |
| Prepared by | GGT (cost code 4034-033) |
| Reviewed by | ________________________ |
| Approved by | ________________________ |

# 1  PURPOSE / SCOPE

This job safety and environmental analysis covers the task plant survey planning, permitting and execution in 203  Plant Surveys at the Mt. Morgan processing plant. It identifies the hazards at each step of the task, the controls required, and the residual risk once those controls are in place.

- Plan and run a plant survey so the data is usable and nobody gets hurt getting it.

- A survey puts people at sample points all over the plant at once, often at height and often in cyanide areas, at a time when the plant is deliberately held steady. It needs planning, not improvisation.

It is read with work instruction SWI-PRO-MET-203-001, which gives the step-by-step method, and with HM-PRC-VXX-PRO207 Plant Surveys and Circuit Performance Assessment, the procedure this task sits under.

# 2  RESPONSIBILITIES

| Role | Responsibilities |
| --- | --- |
| Operator / Metallurgical Technician | Only carry out this task where competent and authorised to do so. Wear the PPE listed in Section 5 without exception. Stop the task and report immediately if any control in Section 4 is not in place. Record the work accurately. Never invent or back-fill a result that was not obtained. |
| Supervisor / Coordinator | Ensure the safety of all personnel carrying out this task. Understand that much of the plant is automated and equipment may start or stop without warning. Confirm the task is authorised and that the area is fit before it starts. Ensure this JSEA is communicated to everyone doing the task and that the sign-on sheet is completed. |
| Manager / Superintendent | Ensure all personnel have received adequate training and are competent. Ensure this JSEA is reviewed at the interval in Section 10 and after any incident or change. Approve the risk ranking in Section 4. |

# 3  ABBREVIATIONS / DEFINITIONS

| Term | Meaning |
| --- | --- |
| AVR | Acidification, volatilisation and regeneration - the cyanide recovery step in Area 081 |
| CIL | Carbon in leach |
| CN Free | Free cyanide, determined by silver nitrate titration |
| DO | Dissolved oxygen |
| HCN | Hydrogen cyanide gas |
| JSEA | Job safety and environmental analysis - this document |
| P80 | The screen size through which 80 per cent of the material passes |
| PPE | Personal protective equipment |
| ReCYN | The resin-in-pulp cyanide and metal recovery process used in Areas 041, 051, 071 and 081 |
| SWI | Standard work instruction - the step-by-step document that sits under this JSEA |
| SWP | Standard work procedure - the circuit-level procedure this task sits under |
| TSF | Tailings storage facility |
| WAD CN | Weak acid dissociable cyanide |

# 4  POTENTIAL HAZARDS

| Scale | Values |
| --- | --- |
| Probability | 1 Rare 2 Unlikely 3 Possible 4 Likely 5 Almost certain |
| Consequence | 1 Insignificant 2 Minor 3 Moderate 4 Major 5 Catastrophic |
| Risk rank | Probability x Consequence. 1-4 Low 5-9 Medium 10-14 High 15-25 Extreme. Scored after the controls in the same row are in place. |

| Job step | Hazards identified | Controls required | P | C | Risk | Actioned by |
| --- | --- | --- | --- | --- | --- | --- |
| 1. Write the survey plan before anything else - objective, points, number of increments, timing, who is where, and what analyses are wanted. | Hydrogen cyanide gas released at the open sample point Working alone in a cyanide area where a collapse would not be seen | Personal HCN monitor worn and switched on, in calibration date. Fixed detection confirmed healthy - alarm 5 ppm, high-high 10 ppm. Sample point approached from upwind. pH of the stream confirmed above 10.5 before opening. Second person in sight and clear. Cyanide antidote kit on site and a trained first aider on shift. Withdraw upwind and report on any reading above the site action level. Cyanide-area sampling is not done alone. Control room notified at the start and end of every round. Radio check-in at the interval set by the Shift Supervisor. | 3 | 5 | 15 EXTREME | Metallurgist |
| 2. Agree the survey window with the Shift Supervisor and the control room. The plant is held at steady state for the survey and nothing is changed during it. | Working alone in a cyanide area where a collapse would not be seen Non-representative sample leading to a wrong process decision | Cyanide-area sampling is not done alone. Control room notified at the start and end of every round. Radio check-in at the interval set by the Shift Supervisor. Line flushed for the stated time before collecting. Sample taken at the same point, in the same way and at the same frequency every time. Container labelled BEFORE the sample is taken, with point, date, time and shift. Steady-state confirmed for 15 to 20 minutes before sampling. A sample taken off-condition is marked as such or discarded. | 3 | 4 | 12 HIGH | Metallurgist |
| 3. Raise the permits the survey needs - working at height, and any restricted access. | Working alone in a cyanide area where a collapse would not be seen | Cyanide-area sampling is not done alone. Control room notified at the start and end of every round. Radio check-in at the interval set by the Shift Supervisor. | 3 | 4 | 12 HIGH | Metallurgist |
| 4. Brief every person on the survey: their points, their route, the timing, the hazards and the stop conditions. | Hydrogen cyanide gas released at the open sample point Fall from a tank top, launder platform, column platform or access ladder Working alone in a cyanide area where a collapse would not be seen | Personal HCN monitor worn and switched on, in calibration date. Fixed detection confirmed healthy - alarm 5 ppm, high-high 10 ppm. Sample point approached from upwind. pH of the stream confirmed above 10.5 before opening. Second person in sight and clear. Cyanide antidote kit on site and a trained first aider on shift. Withdraw upwind and report on any reading above the site action level. Fixed platforms, stairways and handrails used in preference to any other access. Harness and fall arrest where no fixed platform exists. Three points of contact on ladders. No sampling at height alone. Sample containers carried in a bag or hoisted, not in the hand on a ladder. Cyanide-area sampling is not done alone. Control room notified at the start and end of every round. Radio check-in at the interval set by the Shift Supervisor. | 3 | 5 | 15 EXTREME | Metallurgist |
| 5. Pre-label every container before the survey starts. Labelling during the survey wastes the window and causes swaps. | Working alone in a cyanide area where a collapse would not be seen Contact with an agitator, sample cutter, pump or screen that starts without warning Non-representative sample leading to a wrong process decision | Cyanide-area sampling is not done alone. Control room notified at the start and end of every round. Radio check-in at the interval set by the Shift Supervisor. Much of the plant is automated and equipment can start or stop without warning. Guards confirmed in place before approaching. Control room notified before the round starts and again on completion. Isolation and lock-out before any guard or cutter housing is opened. Line flushed for the stated time before collecting. Sample taken at the same point, in the same way and at the same frequency every time. Container labelled BEFORE the sample is taken, with point, date, time and shift. Steady-state confirmed for 15 to 20 minutes before sampling. A sample taken off-condition is marked as such or discarded. | 3 | 5 | 15 EXTREME | Metallurgist |
| 6. Run the survey to the timing in the plan. Take increments at the stated interval, not when convenient. | Working alone in a cyanide area where a collapse would not be seen | Cyanide-area sampling is not done alone. Control room notified at the start and end of every round. Radio check-in at the interval set by the Shift Supervisor. | 3 | 4 | 12 HIGH | Metallurgist |
| 7. Record the plant conditions throughout - tonnage, densities, dosing rates, levels - not just at the start. | Contact with an agitator, sample cutter, pump or screen that starts without warning Non-representative sample leading to a wrong process decision | Much of the plant is automated and equipment can start or stop without warning. Guards confirmed in place before approaching. Control room notified before the round starts and again on completion. Isolation and lock-out before any guard or cutter housing is opened. Line flushed for the stated time before collecting. Sample taken at the same point, in the same way and at the same frequency every time. Container labelled BEFORE the sample is taken, with point, date, time and shift. Steady-state confirmed for 15 to 20 minutes before sampling. A sample taken off-condition is marked as such or discarded. | 3 | 5 | 15 EXTREME | Metallurgist |
| 8. Abandon the survey if the plant leaves steady state, and record why. A survey run through an upset is worse than no survey. | Working alone in a cyanide area where a collapse would not be seen Slip, trip or fall on wet or slurry-covered walkways around sample points Non-representative sample leading to a wrong process decision | Cyanide-area sampling is not done alone. Control room notified at the start and end of every round. Radio check-in at the interval set by the Shift Supervisor. Spillage cleaned up immediately and the point left clean. Walkways kept clear of hoses, buckets and containers. Safety boots with sound tread. Report any damaged or missing grating before use. Line flushed for the stated time before collecting. Sample taken at the same point, in the same way and at the same frequency every time. Container labelled BEFORE the sample is taken, with point, date, time and shift. Steady-state confirmed for 15 to 20 minutes before sampling. A sample taken off-condition is marked as such or discarded. | 3 | 4 | 12 HIGH | Metallurgist |
| 9. Debrief, collect all data sheets and samples, and confirm nothing is outstanding before the teams stand down. | Hydrogen cyanide gas released at the open sample point Splash of cyanide-bearing slurry or solution to skin or eyes Working alone in a cyanide area where a collapse would not be seen | Personal HCN monitor worn and switched on, in calibration date. Fixed detection confirmed healthy - alarm 5 ppm, high-high 10 ppm. Sample point approached from upwind. pH of the stream confirmed above 10.5 before opening. Second person in sight and clear. Cyanide antidote kit on site and a trained first aider on shift. Withdraw upwind and report on any reading above the site action level. Sample valve opened slowly with the container positioned first and the body out of the spray path. Face shield, chemical splash goggles, chemical-resistant gloves and apron. Never open a sample valve whose discharge cannot be seen. Safety shower and eyewash proven flowing before the round starts. Cyanide-area sampling is not done alone. Control room notified at the start and end of every round. Radio check-in at the interval set by the Shift Supervisor. | 3 | 5 | 15 EXTREME | Metallurgist |

# 5  PPE

Standard PPE is worn for all normal operations in the Mt. Morgan gold processing plant. The following is required for this task.

- Hard hat

- Safety glasses

- High-visibility long-sleeved shirt and long trousers

- Safety boots

- Gloves appropriate to the task

- Personal HCN gas monitor, calibrated and in test date

- Chemical splash goggles and face shield

- Nitrile rubber gloves

- Nitrile PVC boots

- Chemical resistant suit

- Full or half gas mask with A2B2E2K2P3 cartridge, carried and available for immediate use

- Harness and fall arrest where no fixed platform exists

# 6  PROCEDURE

6.1  Introduction

Warning: many operations on the gold processing plant are automated and equipment may start or stop without warning. It is the operator's responsibility to ensure the safety of all personnel working on the plant at all times.

Points and equipment covered by this task:

| Tag / point | Description |
| --- | --- |
| Whole plant | Any circuit under survey |

Frequency: As required for a performance survey; typically quarterly and after any significant plant change.

6.2  Job steps

1.  Write the survey plan before anything else - objective, points, number of increments, timing, who is where, and what analyses are wanted.

2.  Agree the survey window with the Shift Supervisor and the control room. The plant is held at steady state for the survey and nothing is changed during it.

3.  Raise the permits the survey needs - working at height, and any restricted access.

4.  Brief every person on the survey: their points, their route, the timing, the hazards and the stop conditions.

5.  Pre-label every container before the survey starts. Labelling during the survey wastes the window and causes swaps.

6.  Run the survey to the timing in the plan. Take increments at the stated interval, not when convenient.

7.  Record the plant conditions throughout - tonnage, densities, dosing rates, levels - not just at the start.

8.  Abandon the survey if the plant leaves steady state, and record why. A survey run through an upset is worse than no survey.

9.  Debrief, collect all data sheets and samples, and confirm nothing is outstanding before the teams stand down.

Equipment required:

- Survey plan and point schedule

- Sample containers pre-labelled by point and time

- Radios for every survey team member

- Field data sheets

- Personal gas monitors for every person

# 7  FLOWCHART

Not reproduced in this document. The step sequence is in work instruction SWI-PRO-MET-203-001 Section 6, which is the controlled version of the method.

# 8  TRAINING

Every person carrying out this task has been through this JSEA at a communication session recorded on the sign-on sheet at the end of this document, and is trained and signed off against work instruction SWI-PRO-MET-203-001.

- Site induction, current.

- Cyanide awareness and gas detection competency where the area requires it.

- Working at height competency where the task requires it.

- Competency in the determinations and calculations carried out in this task.

# 9  REFERENCE

- KBK-MIR-MP-PRO-MET-SOP-0027 Grinding Survey, received 11 Aug 2026 - a worked plant survey end to end

- KBK-MIR-MP-PRO-MET-SOP-0047 Pengukuran Profil Lumpur Cyclone Over Flow, received 11 Aug 2026

- 4034-PR-PRO-002 Sampling Protocol

# 10  REVIEW

This JSEA is reviewed every 12 months, and immediately after any incident, near miss, plant change or change to the schedule affecting this task.

# JSEA COMMUNICATION AND AWARENESS

| Presented by | ________________________ |
| --- | --- |
| Date | ________________________ |
| JSEA reference | JSEA-PRO-MET-203-001 |

| Name | Position | Department | Signature |
| --- | --- | --- | --- |