Mt. Morgan JOB SAFETY AND ENVIRONMENTAL ANALYSIS

Leach and CIL Profile Survey - Tank-by-Tank Gold and Cyanide Profile

Job Safety and Environmental Analysis  ·  Heritage Minerals  ·  Mt. Morgan Processing Plant

| Document number | JSEA-PRO-MET-203-005 |
| --- | --- |
| Work instruction | SWI-PRO-MET-203-005 Leach and CIL Profile Survey - Tank-by-Tank Gold and Cyanide Profile |
| Parent procedure | HM-PRC-VXX-PRO207 Plant Surveys and Circuit Performance Assessment |
| Area | 203 Plant Surveys |
| Classification | Safety-critical |
| Register line | JSEA-PRO-MET-203-005 in HM-MMM-WHS-REG-JSEA-PRO-updated_10, tab JSEA_MET |
| Process owner | Heritage Mineral Operations · Business unit Mt. Morgan |
| Document owner | Process Operations Superintendent |
| Document approver | Process Manager |
| Revision | Rev 0 |
| Date | 11 August 2026 |
| Prepared by | GGT (cost code 4034-033) |
| Reviewed by | ________________________ |
| Approved by | ________________________ |

# 1  PURPOSE / SCOPE

This job safety and environmental analysis covers the task leach and cil profile survey - tank-by-tank gold and cyanide profile in 203  Plant Surveys at the Mt. Morgan processing plant. It identifies the hazards at each step of the task, the controls required, and the residual risk once those controls are in place.

- Build the tank-by-tank gold and cyanide profile down the leach and CIL trains.

- The profile shows where the leach is finishing and where adsorption is actually happening. Without it, carbon is moved and cyanide is dosed on guesswork.

It is read with work instruction SWI-PRO-MET-203-005, which gives the step-by-step method, and with HM-PRC-VXX-PRO207 Plant Surveys and Circuit Performance Assessment, the procedure this task sits under.

# 2  RESPONSIBILITIES

| Role | Responsibilities |
| --- | --- |
| Operator / Metallurgical Technician | Only carry out this task where competent and authorised to do so. Wear the PPE listed in Section 5 without exception. Stop the task and report immediately if any control in Section 4 is not in place. Record the work accurately. Never invent or back-fill a result that was not obtained. |
| Supervisor / Coordinator | Ensure the safety of all personnel carrying out this task. Understand that much of the plant is automated and equipment may start or stop without warning. Confirm the task is authorised and that the area is fit before it starts. Ensure this JSEA is communicated to everyone doing the task and that the sign-on sheet is completed. |
| Manager / Superintendent | Ensure all personnel have received adequate training and are competent. Ensure this JSEA is reviewed at the interval in Section 10 and after any incident or change. Approve the risk ranking in Section 4. |

# 3  ABBREVIATIONS / DEFINITIONS

| Term | Meaning |
| --- | --- |
| AVR | Acidification, volatilisation and regeneration - the cyanide recovery step in Area 081 |
| CIL | Carbon in leach |
| CN Free | Free cyanide, determined by silver nitrate titration |
| DO | Dissolved oxygen |
| HCN | Hydrogen cyanide gas |
| JSEA | Job safety and environmental analysis - this document |
| P80 | The screen size through which 80 per cent of the material passes |
| PPE | Personal protective equipment |
| ReCYN | The resin-in-pulp cyanide and metal recovery process used in Areas 041, 051, 071 and 081 |
| SWI | Standard work instruction - the step-by-step document that sits under this JSEA |
| SWP | Standard work procedure - the circuit-level procedure this task sits under |
| TSF | Tailings storage facility |
| WAD CN | Weak acid dissociable cyanide |

# 4  POTENTIAL HAZARDS

| Scale | Values |
| --- | --- |
| Probability | 1 Rare 2 Unlikely 3 Possible 4 Likely 5 Almost certain |
| Consequence | 1 Insignificant 2 Minor 3 Moderate 4 Major 5 Catastrophic |
| Risk rank | Probability x Consequence. 1-4 Low 5-9 Medium 10-14 High 15-25 Extreme. Scored after the controls in the same row are in place. |

| Job step | Hazards identified | Controls required | P | C | Risk | Actioned by |
| --- | --- | --- | --- | --- | --- | --- |
| 1. Confirm steady state and hold the circuit through the survey. | Working alone in a cyanide area where a collapse would not be seen Non-representative sample leading to a wrong process decision | Cyanide-area sampling is not done alone. Control room notified at the start and end of every round. Radio check-in at the interval set by the Shift Supervisor. Line flushed for the stated time before collecting. Sample taken at the same point, in the same way and at the same frequency every time. Container labelled BEFORE the sample is taken, with point, date, time and shift. Steady-state confirmed for 15 to 20 minutes before sampling. A sample taken off-condition is marked as such or discarded. | 3 | 4 | 12 HIGH | Metallurgist |
| 2. Sample every tank in the train in sequence, in one pass, so the profile represents one moment. | Hydrogen cyanide gas released at the open sample point Fall from a tank top, launder platform, column platform or access ladder Confined space atmosphere at a tank top hatch, launder or hopper opening | Personal HCN monitor worn and switched on, in calibration date. Fixed detection confirmed healthy - alarm 5 ppm, high-high 10 ppm. Sample point approached from upwind. pH of the stream confirmed above 10.5 before opening. Second person in sight and clear. Cyanide antidote kit on site and a trained first aider on shift. Withdraw upwind and report on any reading above the site action level. Fixed platforms, stairways and handrails used in preference to any other access. Harness and fall arrest where no fixed platform exists. Three points of contact on ladders. No sampling at height alone. Sample containers carried in a bag or hoisted, not in the hand on a ladder. No entry. Sampling is done from outside the opening only. Confined space entry, if it is ever needed for a sample point, requires a permit, atmosphere testing, isolation, a standby person and a rescue plan under the site confined space procedure. | 3 | 5 | 15 EXTREME | Metallurgist |
| 3. Determine pH, dissolved oxygen and free cyanide in the field at each tank. | Hydrogen cyanide gas released at the open sample point Fall from a tank top, launder platform, column platform or access ladder Confined space atmosphere at a tank top hatch, launder or hopper opening | Personal HCN monitor worn and switched on, in calibration date. Fixed detection confirmed healthy - alarm 5 ppm, high-high 10 ppm. Sample point approached from upwind. pH of the stream confirmed above 10.5 before opening. Second person in sight and clear. Cyanide antidote kit on site and a trained first aider on shift. Withdraw upwind and report on any reading above the site action level. Fixed platforms, stairways and handrails used in preference to any other access. Harness and fall arrest where no fixed platform exists. Three points of contact on ladders. No sampling at height alone. Sample containers carried in a bag or hoisted, not in the hand on a ladder. No entry. Sampling is done from outside the opening only. Confined space entry, if it is ever needed for a sample point, requires a permit, atmosphere testing, isolation, a standby person and a rescue plan under the site confined space procedure. | 3 | 5 | 15 EXTREME | Metallurgist |
| 4. Filter each sample and submit solution and solids for gold assay. | Hydrogen cyanide gas released at the open sample point Splash of cyanide-bearing slurry or solution to skin or eyes Non-representative sample leading to a wrong process decision | Personal HCN monitor worn and switched on, in calibration date. Fixed detection confirmed healthy - alarm 5 ppm, high-high 10 ppm. Sample point approached from upwind. pH of the stream confirmed above 10.5 before opening. Second person in sight and clear. Cyanide antidote kit on site and a trained first aider on shift. Withdraw upwind and report on any reading above the site action level. Sample valve opened slowly with the container positioned first and the body out of the spray path. Face shield, chemical splash goggles, chemical-resistant gloves and apron. Never open a sample valve whose discharge cannot be seen. Safety shower and eyewash proven flowing before the round starts. Line flushed for the stated time before collecting. Sample taken at the same point, in the same way and at the same frequency every time. Container labelled BEFORE the sample is taken, with point, date, time and shift. Steady-state confirmed for 15 to 20 minutes before sampling. A sample taken off-condition is marked as such or discarded. | 3 | 5 | 15 EXTREME | Metallurgist |
| 5. Take carbon concentration and carbon loading for each CIL tank on the same pass. | Hydrogen cyanide gas released at the open sample point Fall from a tank top, launder platform, column platform or access ladder Confined space atmosphere at a tank top hatch, launder or hopper opening | Personal HCN monitor worn and switched on, in calibration date. Fixed detection confirmed healthy - alarm 5 ppm, high-high 10 ppm. Sample point approached from upwind. pH of the stream confirmed above 10.5 before opening. Second person in sight and clear. Cyanide antidote kit on site and a trained first aider on shift. Withdraw upwind and report on any reading above the site action level. Fixed platforms, stairways and handrails used in preference to any other access. Harness and fall arrest where no fixed platform exists. Three points of contact on ladders. No sampling at height alone. Sample containers carried in a bag or hoisted, not in the hand on a ladder. No entry. Sampling is done from outside the opening only. Confined space entry, if it is ever needed for a sample point, requires a permit, atmosphere testing, isolation, a standby person and a rescue plan under the site confined space procedure. | 3 | 5 | 15 EXTREME | Metallurgist |
| 6. Plot solution gold, solids gold and carbon loading down the train. | Hydrogen cyanide gas released at the open sample point | Personal HCN monitor worn and switched on, in calibration date. Fixed detection confirmed healthy - alarm 5 ppm, high-high 10 ppm. Sample point approached from upwind. pH of the stream confirmed above 10.5 before opening. Second person in sight and clear. Cyanide antidote kit on site and a trained first aider on shift. Withdraw upwind and report on any reading above the site action level. | 3 | 5 | 15 EXTREME | Metallurgist |
| 7. Identify the tank where leaching finishes and the tank where adsorption stops being effective. | Hydrogen cyanide gas released at the open sample point Fall from a tank top, launder platform, column platform or access ladder Confined space atmosphere at a tank top hatch, launder or hopper opening | Personal HCN monitor worn and switched on, in calibration date. Fixed detection confirmed healthy - alarm 5 ppm, high-high 10 ppm. Sample point approached from upwind. pH of the stream confirmed above 10.5 before opening. Second person in sight and clear. Cyanide antidote kit on site and a trained first aider on shift. Withdraw upwind and report on any reading above the site action level. Fixed platforms, stairways and handrails used in preference to any other access. Harness and fall arrest where no fixed platform exists. Three points of contact on ladders. No sampling at height alone. Sample containers carried in a bag or hoisted, not in the hand on a ladder. No entry. Sampling is done from outside the opening only. Confined space entry, if it is ever needed for a sample point, requires a permit, atmosphere testing, isolation, a standby person and a rescue plan under the site confined space procedure. | 3 | 5 | 15 EXTREME | Metallurgist |
| 8. Report against the design profile and recommend any carbon movement or dosing change. | Non-representative sample leading to a wrong process decision | Line flushed for the stated time before collecting. Sample taken at the same point, in the same way and at the same frequency every time. Container labelled BEFORE the sample is taken, with point, date, time and shift. Steady-state confirmed for 15 to 20 minutes before sampling. A sample taken off-condition is marked as such or discarded. | 3 | 3 | 9 MEDIUM | Metallurgist |

# 5  PPE

Standard PPE is worn for all normal operations in the Mt. Morgan gold processing plant. The following is required for this task.

- Hard hat

- Safety glasses

- High-visibility long-sleeved shirt and long trousers

- Safety boots

- Gloves appropriate to the task

- Personal HCN gas monitor, calibrated and in test date

- Chemical splash goggles and face shield

- Nitrile rubber gloves

- Nitrile PVC boots

- Chemical resistant suit

- Full or half gas mask with A2B2E2K2P3 cartridge, carried and available for immediate use

- Harness and fall arrest where no fixed platform exists

# 6  PROCEDURE

6.1  Introduction

Warning: many operations on the gold processing plant are automated and equipment may start or stop without warning. It is the operator's responsibility to ensure the safety of all personnel working on the plant at all times.

Points and equipment covered by this task:

| Tag / point | Description |
| --- | --- |
| 031-TK-001 | Leach tank |
| 032-TK-002 to -006 | CIL tanks 1 to 5 |

Frequency: Quarterly, and whenever recovery moves without an obvious cause.

6.2  Job steps

1.  Confirm steady state and hold the circuit through the survey.

2.  Sample every tank in the train in sequence, in one pass, so the profile represents one moment.

3.  Determine pH, dissolved oxygen and free cyanide in the field at each tank.

4.  Filter each sample and submit solution and solids for gold assay.

5.  Take carbon concentration and carbon loading for each CIL tank on the same pass.

6.  Plot solution gold, solids gold and carbon loading down the train.

7.  Identify the tank where leaching finishes and the tank where adsorption stops being effective.

8.  Report against the design profile and recommend any carbon movement or dosing change.

Equipment required:

- Sample containers pre-labelled by tank

- pH meter

- DO meter

- Free cyanide titration kit

- Filter press and filter paper

- Field data sheets

# 7  FLOWCHART

Not reproduced in this document. The step sequence is in work instruction SWI-PRO-MET-203-005 Section 6, which is the controlled version of the method.

# 8  TRAINING

Every person carrying out this task has been through this JSEA at a communication session recorded on the sign-on sheet at the end of this document, and is trained and signed off against work instruction SWI-PRO-MET-203-005.

- Site induction, current.

- Cyanide awareness and gas detection competency where the area requires it.

- Working at height competency where the task requires it.

- Competency in the determinations and calculations carried out in this task.

# 9  REFERENCE

- 4034-PR-PRO-002 Sampling Protocol, Leach Area table

- KBK-MIR-MP-PRO-MET-SOP-0019 Profil Leaching and SOP-0048 leach tank sampling and profile, both received 11 Aug 2026

- HM-PRC-V01-PRO004 Leach Circuit and PRO005 Carbon in Leach

# 10  REVIEW

This JSEA is reviewed every 12 months, and immediately after any incident, near miss, plant change or change to the schedule affecting this task.

# JSEA COMMUNICATION AND AWARENESS

| Presented by | ________________________ |
| --- | --- |
| Date | ________________________ |
| JSEA reference | JSEA-PRO-MET-203-005 |

| Name | Position | Department | Signature |
| --- | --- | --- | --- |