Mt. Morgan JOB SAFETY AND ENVIRONMENTAL ANALYSIS

Oxygen Utilisation and Sparger Performance Survey

Job Safety and Environmental Analysis  ·  Heritage Minerals  ·  Mt. Morgan Processing Plant

| Document number | JSEA-PRO-MET-203-012 |
| --- | --- |
| Work instruction | SWI-PRO-MET-203-012 Oxygen Utilisation and Sparger Performance Survey |
| Parent procedure | HM-PRC-VXX-PRO207 Plant Surveys and Circuit Performance Assessment |
| Area | 203 Plant Surveys |
| Classification | Safety-critical |
| Register line | JSEA-PRO-MET-203-012 in HM-MMM-WHS-REG-JSEA-PRO-updated_10, tab JSEA_MET |
| Process owner | Heritage Mineral Operations · Business unit Mt. Morgan |
| Document owner | Process Operations Superintendent |
| Document approver | Process Manager |
| Revision | Rev 0 |
| Date | 11 August 2026 |
| Prepared by | GGT (cost code 4034-033) |
| Reviewed by | ________________________ |
| Approved by | ________________________ |

# 1  PURPOSE / SCOPE

This job safety and environmental analysis covers the task oxygen utilisation and sparger performance survey in 203  Plant Surveys at the Mt. Morgan processing plant. It identifies the hazards at each step of the task, the controls required, and the residual risk once those controls are in place.

- Measure how much of the oxygen injected into the leach is actually being used, and whether the spargers are delivering it.

- Oxygen is a direct input to gold dissolution. A blocked or worn sparger costs recovery quietly.

It is read with work instruction SWI-PRO-MET-203-012, which gives the step-by-step method, and with HM-PRC-VXX-PRO207 Plant Surveys and Circuit Performance Assessment, the procedure this task sits under.

# 2  RESPONSIBILITIES

| Role | Responsibilities |
| --- | --- |
| Operator / Metallurgical Technician | Only carry out this task where competent and authorised to do so. Wear the PPE listed in Section 5 without exception. Stop the task and report immediately if any control in Section 4 is not in place. Record the work accurately. Never invent or back-fill a result that was not obtained. |
| Supervisor / Coordinator | Ensure the safety of all personnel carrying out this task. Understand that much of the plant is automated and equipment may start or stop without warning. Confirm the task is authorised and that the area is fit before it starts. Ensure this JSEA is communicated to everyone doing the task and that the sign-on sheet is completed. |
| Manager / Superintendent | Ensure all personnel have received adequate training and are competent. Ensure this JSEA is reviewed at the interval in Section 10 and after any incident or change. Approve the risk ranking in Section 4. |

# 3  ABBREVIATIONS / DEFINITIONS

| Term | Meaning |
| --- | --- |
| AVR | Acidification, volatilisation and regeneration - the cyanide recovery step in Area 081 |
| CIL | Carbon in leach |
| CN Free | Free cyanide, determined by silver nitrate titration |
| DO | Dissolved oxygen |
| HCN | Hydrogen cyanide gas |
| JSEA | Job safety and environmental analysis - this document |
| P80 | The screen size through which 80 per cent of the material passes |
| PPE | Personal protective equipment |
| ReCYN | The resin-in-pulp cyanide and metal recovery process used in Areas 041, 051, 071 and 081 |
| SWI | Standard work instruction - the step-by-step document that sits under this JSEA |
| SWP | Standard work procedure - the circuit-level procedure this task sits under |
| TSF | Tailings storage facility |
| WAD CN | Weak acid dissociable cyanide |

# 4  POTENTIAL HAZARDS

| Scale | Values |
| --- | --- |
| Probability | 1 Rare 2 Unlikely 3 Possible 4 Likely 5 Almost certain |
| Consequence | 1 Insignificant 2 Minor 3 Moderate 4 Major 5 Catastrophic |
| Risk rank | Probability x Consequence. 1-4 Low 5-9 Medium 10-14 High 15-25 Extreme. Scored after the controls in the same row are in place. |

| Job step | Hazards identified | Controls required | P | C | Risk | Actioned by |
| --- | --- | --- | --- | --- | --- | --- |
| 1. Record the oxygen supply pressure and flow to each sparger from the control system. | Release from a pressurised sample line, hose or fitting Contact with an agitator, sample cutter, pump or screen that starts without warning Non-representative sample leading to a wrong process decision | Confirm the sample line is not blocked before opening. Open slowly and stand clear of the discharge path. Report any weeping fitting rather than tightening it under pressure. Much of the plant is automated and equipment can start or stop without warning. Guards confirmed in place before approaching. Control room notified before the round starts and again on completion. Isolation and lock-out before any guard or cutter housing is opened. Line flushed for the stated time before collecting. Sample taken at the same point, in the same way and at the same frequency every time. Container labelled BEFORE the sample is taken, with point, date, time and shift. Steady-state confirmed for 15 to 20 minutes before sampling. A sample taken off-condition is marked as such or discarded. | 3 | 5 | 15 EXTREME | Metallurgist |
| 2. Measure dissolved oxygen at several depths and positions in the tank, not at one point. A single reading at the launder does not describe the tank. | Hydrogen cyanide gas released at the open sample point Fall from a tank top, launder platform, column platform or access ladder | Personal HCN monitor worn and switched on, in calibration date. Fixed detection confirmed healthy - alarm 5 ppm, high-high 10 ppm. Sample point approached from upwind. pH of the stream confirmed above 10.5 before opening. Second person in sight and clear. Cyanide antidote kit on site and a trained first aider on shift. Withdraw upwind and report on any reading above the site action level. Fixed platforms, stairways and handrails used in preference to any other access. Harness and fall arrest where no fixed platform exists. Three points of contact on ladders. No sampling at height alone. Sample containers carried in a bag or hoisted, not in the hand on a ladder. | 3 | 5 | 15 EXTREME | Metallurgist |
| 3. Take the DO reading immediately on sampling - atmospheric exchange starts as soon as the sample is drawn. | Hydrogen cyanide gas released at the open sample point Contact with an agitator, sample cutter, pump or screen that starts without warning Non-representative sample leading to a wrong process decision | Personal HCN monitor worn and switched on, in calibration date. Fixed detection confirmed healthy - alarm 5 ppm, high-high 10 ppm. Sample point approached from upwind. pH of the stream confirmed above 10.5 before opening. Second person in sight and clear. Cyanide antidote kit on site and a trained first aider on shift. Withdraw upwind and report on any reading above the site action level. Much of the plant is automated and equipment can start or stop without warning. Guards confirmed in place before approaching. Control room notified before the round starts and again on completion. Isolation and lock-out before any guard or cutter housing is opened. Line flushed for the stated time before collecting. Sample taken at the same point, in the same way and at the same frequency every time. Container labelled BEFORE the sample is taken, with point, date, time and shift. Steady-state confirmed for 15 to 20 minutes before sampling. A sample taken off-condition is marked as such or discarded. | 3 | 5 | 15 EXTREME | Metallurgist |
| 4. Calculate oxygen utilisation from the injected flow and the measured dissolved oxygen against the consumption implied by the leach rate. | Non-representative sample leading to a wrong process decision | Line flushed for the stated time before collecting. Sample taken at the same point, in the same way and at the same frequency every time. Container labelled BEFORE the sample is taken, with point, date, time and shift. Steady-state confirmed for 15 to 20 minutes before sampling. A sample taken off-condition is marked as such or discarded. | 3 | 3 | 9 MEDIUM | Metallurgist |
| 5. Compare the sparger operating pressure against the Hypersparge instructions. A sparger operating below its design pressure is not atomising. | Release from a pressurised sample line, hose or fitting Contact with an agitator, sample cutter, pump or screen that starts without warning | Confirm the sample line is not blocked before opening. Open slowly and stand clear of the discharge path. Report any weeping fitting rather than tightening it under pressure. Much of the plant is automated and equipment can start or stop without warning. Guards confirmed in place before approaching. Control room notified before the round starts and again on completion. Isolation and lock-out before any guard or cutter housing is opened. | 2 | 5 | 10 HIGH | Metallurgist |
| 6. Inspect the sparger condition against the vendor criteria where access allows. | Hydrogen cyanide gas released at the open sample point | Personal HCN monitor worn and switched on, in calibration date. Fixed detection confirmed healthy - alarm 5 ppm, high-high 10 ppm. Sample point approached from upwind. pH of the stream confirmed above 10.5 before opening. Second person in sight and clear. Cyanide antidote kit on site and a trained first aider on shift. Withdraw upwind and report on any reading above the site action level. | 3 | 5 | 15 EXTREME | Metallurgist |
| 7. Report utilisation, the DO profile and any sparger due for cleaning or replacement. | Non-representative sample leading to a wrong process decision | Line flushed for the stated time before collecting. Sample taken at the same point, in the same way and at the same frequency every time. Container labelled BEFORE the sample is taken, with point, date, time and shift. Steady-state confirmed for 15 to 20 minutes before sampling. A sample taken off-condition is marked as such or discarded. | 3 | 3 | 9 MEDIUM | Metallurgist |

# 5  PPE

Standard PPE is worn for all normal operations in the Mt. Morgan gold processing plant. The following is required for this task.

- Hard hat

- Safety glasses

- High-visibility long-sleeved shirt and long trousers

- Safety boots

- Gloves appropriate to the task

- Personal HCN gas monitor, calibrated and in test date

- Chemical splash goggles and face shield

- Nitrile rubber gloves

- Nitrile PVC boots

- Chemical resistant suit

- Full or half gas mask with A2B2E2K2P3 cartridge, carried and available for immediate use

- Harness and fall arrest where no fixed platform exists

# 6  PROCEDURE

6.1  Introduction

Warning: many operations on the gold processing plant are automated and equipment may start or stop without warning. It is the operator's responsibility to ensure the safety of all personnel working on the plant at all times.

Points and equipment covered by this task:

| Tag / point | Description |
| --- | --- |
| 031-TK-001 | Leach tank - oxygen spargers |
| Hypersparge | Supersonic gas spargers DN15 and DN25 |

Frequency: Quarterly, and after any sparger clean or replacement.

6.2  Job steps

1.  Record the oxygen supply pressure and flow to each sparger from the control system.

2.  Measure dissolved oxygen at several depths and positions in the tank, not at one point. A single reading at the launder does not describe the tank.

3.  Take the DO reading immediately on sampling - atmospheric exchange starts as soon as the sample is drawn.

4.  Calculate oxygen utilisation from the injected flow and the measured dissolved oxygen against the consumption implied by the leach rate.

5.  Compare the sparger operating pressure against the Hypersparge instructions. A sparger operating below its design pressure is not atomising.

6.  Inspect the sparger condition against the vendor criteria where access allows.

7.  Report utilisation, the DO profile and any sparger due for cleaning or replacement.

Equipment required:

- DO meter, calibrated

- Sample containers

- Stopwatch

- Field data sheets

- Oxygen flow readings from the control system

# 7  FLOWCHART

Not reproduced in this document. The step sequence is in work instruction SWI-PRO-MET-203-012 Section 6, which is the controlled version of the method.

# 8  TRAINING

Every person carrying out this task has been through this JSEA at a communication session recorded on the sign-on sheet at the end of this document, and is trained and signed off against work instruction SWI-PRO-MET-203-012.

- Site induction, current.

- Cyanide awareness and gas detection competency where the area requires it.

- Working at height competency where the task requires it.

- Competency in the determinations and calculations carried out in this task.

# 9  REFERENCE

- 4034-044-VE-003 Rev B Glencore Hypersparge supersonic gas sparger installation and operating instructions, received 11 Aug 2026

- KBK-MIR-MP-PRO-MET-SOP-0046 Pengukuran Oksigen Terlarut, received 11 Aug 2026

- HM-PRC-V01-PRO004 Leach Circuit and SWI-PRO-OPR-031-004 Oxygen Sparger Inspection and Cleaning

# 10  REVIEW

This JSEA is reviewed every 12 months, and immediately after any incident, near miss, plant change or change to the schedule affecting this task.

# JSEA COMMUNICATION AND AWARENESS

| Presented by | ________________________ |
| --- | --- |
| Date | ________________________ |
| JSEA reference | JSEA-PRO-MET-203-012 |

| Name | Position | Department | Signature |
| --- | --- | --- | --- |