Mt. Morgan JOB SAFETY AND ENVIRONMENTAL ANALYSIS

Wet Screening and Sizing Analysis

Job Safety and Environmental Analysis  ·  Heritage Minerals  ·  Mt. Morgan Processing Plant

| Document number | JSEA-PRO-MET-204-002 |
| --- | --- |
| Work instruction | SWI-PRO-MET-204-002 Wet Screening and Sizing Analysis |
| Parent procedure | HM-PRC-VXX-PRO208 Metallurgical Laboratory Physical Testwork |
| Area | 204 Met Lab - Physical Testwork |
| Classification | Operating |
| Register line | JSEA-PRO-MET-204-002 in HM-MMM-WHS-REG-JSEA-PRO-updated_10, tab JSEA_MET |
| Process owner | Heritage Mineral Operations · Business unit Mt. Morgan |
| Document owner | Process Operations Superintendent |
| Document approver | Process Manager |
| Revision | Rev 0 |
| Date | 11 August 2026 |
| Prepared by | GGT (cost code 4034-033) |
| Reviewed by | ________________________ |
| Approved by | ________________________ |

# 1  PURPOSE / SCOPE

This job safety and environmental analysis covers the task wet screening and sizing analysis in 204  Met Lab - Physical Testwork at the Mt. Morgan processing plant. It identifies the hazards at each step of the task, the controls required, and the residual risk once those controls are in place.

- Determine the size distribution of a slurry sample by wet screening.

- Sizing drives grind control, cyclone assessment and every liberation question that follows.

It is read with work instruction SWI-PRO-MET-204-002, which gives the step-by-step method, and with HM-PRC-VXX-PRO208 Metallurgical Laboratory Physical Testwork, the procedure this task sits under.

# 2  RESPONSIBILITIES

| Role | Responsibilities |
| --- | --- |
| Operator / Metallurgical Technician | Only carry out this task where competent and authorised to do so. Wear the PPE listed in Section 5 without exception. Stop the task and report immediately if any control in Section 4 is not in place. Record the work accurately. Never invent or back-fill a result that was not obtained. |
| Supervisor / Coordinator | Ensure the safety of all personnel carrying out this task. Understand that much of the plant is automated and equipment may start or stop without warning. Confirm the task is authorised and that the area is fit before it starts. Ensure this JSEA is communicated to everyone doing the task and that the sign-on sheet is completed. |
| Manager / Superintendent | Ensure all personnel have received adequate training and are competent. Ensure this JSEA is reviewed at the interval in Section 10 and after any incident or change. Approve the risk ranking in Section 4. |

# 3  ABBREVIATIONS / DEFINITIONS

| Term | Meaning |
| --- | --- |
| AVR | Acidification, volatilisation and regeneration - the cyanide recovery step in Area 081 |
| CIL | Carbon in leach |
| CN Free | Free cyanide, determined by silver nitrate titration |
| DO | Dissolved oxygen |
| HCN | Hydrogen cyanide gas |
| JSEA | Job safety and environmental analysis - this document |
| P80 | The screen size through which 80 per cent of the material passes |
| PPE | Personal protective equipment |
| ReCYN | The resin-in-pulp cyanide and metal recovery process used in Areas 041, 051, 071 and 081 |
| SWI | Standard work instruction - the step-by-step document that sits under this JSEA |
| SWP | Standard work procedure - the circuit-level procedure this task sits under |
| TSF | Tailings storage facility |
| WAD CN | Weak acid dissociable cyanide |

# 4  POTENTIAL HAZARDS

| Scale | Values |
| --- | --- |
| Probability | 1 Rare 2 Unlikely 3 Possible 4 Likely 5 Almost certain |
| Consequence | 1 Insignificant 2 Minor 3 Moderate 4 Major 5 Catastrophic |
| Risk rank | Probability x Consequence. 1-4 Low 5-9 Medium 10-14 High 15-25 Extreme. Scored after the controls in the same row are in place. |

| Job step | Hazards identified | Controls required | P | C | Risk | Actioned by |
| --- | --- | --- | --- | --- | --- | --- |
| 1. Weigh the wet sample and record the receipt mass. | Inhalation of or contact with solid reagent - cyanide briquettes, lime or salt Non-representative sample leading to a wrong process decision | Sampling of solid cyanide done only with the full cyanide PPE set including a gas mask with an A2B2E2K2P3 cartridge. Dust suppression and local extraction confirmed running. No dry sweeping. Line flushed for the stated time before collecting. Sample taken at the same point, in the same way and at the same frequency every time. Container labelled BEFORE the sample is taken, with point, date, time and shift. Steady-state confirmed for 15 to 20 minutes before sampling. A sample taken off-condition is marked as such or discarded. | 3 | 4 | 12 HIGH | Met Lab Technician |
| 2. Wet screen the sample at the finest aperture to remove the undersize, washing until the water runs clear. | Hearing damage in the mill, screen, compressor and blower areas Contact with an agitator, sample cutter, pump or screen that starts without warning Non-representative sample leading to a wrong process decision | Hearing protection mandatory in signed areas. Double protection at the mill and compressor. Time at the point kept to what the task needs. Much of the plant is automated and equipment can start or stop without warning. Guards confirmed in place before approaching. Control room notified before the round starts and again on completion. Isolation and lock-out before any guard or cutter housing is opened. Line flushed for the stated time before collecting. Sample taken at the same point, in the same way and at the same frequency every time. Container labelled BEFORE the sample is taken, with point, date, time and shift. Steady-state confirmed for 15 to 20 minutes before sampling. A sample taken off-condition is marked as such or discarded. | 3 | 5 | 15 EXTREME | Met Lab Technician |
| 3. Dry both the oversize and the recovered undersize to constant mass. | Inhalation of or contact with solid reagent - cyanide briquettes, lime or salt | Sampling of solid cyanide done only with the full cyanide PPE set including a gas mask with an A2B2E2K2P3 cartridge. Dust suppression and local extraction confirmed running. No dry sweeping. | 3 | 4 | 12 HIGH | Met Lab Technician |
| 4. Dry screen the oversize through the full sieve set on the Ro-Tap for the standard period. | Inhalation of or contact with solid reagent - cyanide briquettes, lime or salt Hearing damage in the mill, screen, compressor and blower areas Contact with an agitator, sample cutter, pump or screen that starts without warning | Sampling of solid cyanide done only with the full cyanide PPE set including a gas mask with an A2B2E2K2P3 cartridge. Dust suppression and local extraction confirmed running. No dry sweeping. Hearing protection mandatory in signed areas. Double protection at the mill and compressor. Time at the point kept to what the task needs. Much of the plant is automated and equipment can start or stop without warning. Guards confirmed in place before approaching. Control room notified before the round starts and again on completion. Isolation and lock-out before any guard or cutter housing is opened. | 3 | 5 | 15 EXTREME | Met Lab Technician |
| 5. Weigh each retained fraction and the pan. | Inhalation of or contact with solid reagent - cyanide briquettes, lime or salt | Sampling of solid cyanide done only with the full cyanide PPE set including a gas mask with an A2B2E2K2P3 cartridge. Dust suppression and local extraction confirmed running. No dry sweeping. | 3 | 4 | 12 HIGH | Met Lab Technician |
| 6. Check the sum of fractions against the dry receipt mass. A loss above the allowed tolerance means the screening is repeated, not reported. | Hearing damage in the mill, screen, compressor and blower areas Contact with an agitator, sample cutter, pump or screen that starts without warning Non-representative sample leading to a wrong process decision | Hearing protection mandatory in signed areas. Double protection at the mill and compressor. Time at the point kept to what the task needs. Much of the plant is automated and equipment can start or stop without warning. Guards confirmed in place before approaching. Control room notified before the round starts and again on completion. Isolation and lock-out before any guard or cutter housing is opened. Line flushed for the stated time before collecting. Sample taken at the same point, in the same way and at the same frequency every time. Container labelled BEFORE the sample is taken, with point, date, time and shift. Steady-state confirmed for 15 to 20 minutes before sampling. A sample taken off-condition is marked as such or discarded. | 3 | 5 | 15 EXTREME | Met Lab Technician |
| 7. Calculate the cumulative passing distribution and the P80. | Non-representative sample leading to a wrong process decision | Line flushed for the stated time before collecting. Sample taken at the same point, in the same way and at the same frequency every time. Container labelled BEFORE the sample is taken, with point, date, time and shift. Steady-state confirmed for 15 to 20 minutes before sampling. A sample taken off-condition is marked as such or discarded. | 3 | 3 | 9 MEDIUM | Met Lab Technician |
| 8. Record the result with the sieve set identity and the shaking period. | Inhalation of or contact with solid reagent - cyanide briquettes, lime or salt Cross-contamination from a container or cutter not cleaned between points Non-representative sample leading to a wrong process decision | Sampling of solid cyanide done only with the full cyanide PPE set including a gas mask with an A2B2E2K2P3 cartridge. Dust suppression and local extraction confirmed running. No dry sweeping. Bucket, container, cutter and sieve rinsed with process water and then with the stream being sampled, and the rinse discarded, before every sample. Dedicated containers for high-grade streams. Labels checked against the schedule before handover. Line flushed for the stated time before collecting. Sample taken at the same point, in the same way and at the same frequency every time. Container labelled BEFORE the sample is taken, with point, date, time and shift. Steady-state confirmed for 15 to 20 minutes before sampling. A sample taken off-condition is marked as such or discarded. | 3 | 4 | 12 HIGH | Met Lab Technician |

# 5  PPE

Standard PPE is worn for all normal operations in the Mt. Morgan gold processing plant. The following is required for this task.

- Hard hat

- Safety glasses

- High-visibility long-sleeved shirt and long trousers

- Safety boots

- Gloves appropriate to the task

- Personal HCN gas monitor, calibrated and in test date

- Chemical splash goggles and face shield

- Nitrile rubber gloves

- Nitrile PVC boots

- Chemical resistant suit

- Full or half gas mask with A2B2E2K2P3 cartridge, carried and available for immediate use

# 6  PROCEDURE

6.1  Introduction

Warning: many operations on the gold processing plant are automated and equipment may start or stop without warning. It is the operator's responsibility to ensure the safety of all personnel working on the plant at all times.

Points and equipment covered by this task:

| Tag / point | Description |
| --- | --- |
| Met lab bench | Physical testwork bench and instrument |

Frequency: Per the sampling schedule; daily on CIL tail and cyclone overflow.

6.2  Job steps

1.  Weigh the wet sample and record the receipt mass.

2.  Wet screen the sample at the finest aperture to remove the undersize, washing until the water runs clear.

3.  Dry both the oversize and the recovered undersize to constant mass.

4.  Dry screen the oversize through the full sieve set on the Ro-Tap for the standard period.

5.  Weigh each retained fraction and the pan.

6.  Check the sum of fractions against the dry receipt mass. A loss above the allowed tolerance means the screening is repeated, not reported.

7.  Calculate the cumulative passing distribution and the P80.

8.  Record the result with the sieve set identity and the shaking period.

Equipment required:

- Sieve set 38 um to 2 mm

- Ro-Tap RX29 shaker and sound enclosure

- Wash bottle and spray

- Drying oven

- Balance 0.1 mg

- Drying trays

# 7  FLOWCHART

Not reproduced in this document. The step sequence is in work instruction SWI-PRO-MET-204-002 Section 6, which is the controlled version of the method.

# 8  TRAINING

Every person carrying out this task has been through this JSEA at a communication session recorded on the sign-on sheet at the end of this document, and is trained and signed off against work instruction SWI-PRO-MET-204-002.

- Site induction, current.

- Cyanide awareness and gas detection competency where the area requires it.

- Working at height competency where the task requires it.

- Competency in the determinations and calculations carried out in this task.

# 9  REFERENCE

- KBK-MIR-LAB-PRE-SOP-008 Uji Kehalusan Sampel

- KBK-MIR-MP-PRO-MET-SOP-0020 Distribusi Logam dan Ukuran, SOP-0028 Sizing Harian CIL Tail and SOP-0029 Sizing Harian COF, all received 11 Aug 2026

- ILS quotation Q0023979 - Ro-Tap RX29, sound enclosure and twelve 200 mm sieves 38 um to 2 mm

# 10  REVIEW

This JSEA is reviewed every 12 months, and immediately after any incident, near miss, plant change or change to the schedule affecting this task.

# JSEA COMMUNICATION AND AWARENESS

| Presented by | ________________________ |
| --- | --- |
| Date | ________________________ |
| JSEA reference | JSEA-PRO-MET-204-002 |

| Name | Position | Department | Signature |
| --- | --- | --- | --- |