| Company: | Heritage Minerals Pty Ltd |  |  |  |  |  |  |  |  |  |  |  | ABN: |  |  |  | 61 633 121 954 |  |  |  |  |  |  |  |  |  |  | Date Developed: |  |  |  |  | 11-08-2026 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operation: | Mount Morgan Gold Mine |  |  |  |  |  |  |  |  |  |  |  | Supervisor: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Review Period: |  |  |  |  | 12 Months |  |  |  |  |
| Work Activity: | Cyanide Destruction (Detox) Bench Test (205 Met Lab - Chemical and Leach Testwork) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | JSEA/SWMS No.: |  |  |  |  | HM-MMM-WHS-FRM-031-R03-JSEA-PRO-MET-205-015 |  |  |  |  |
| PERSONAL PROTECTIVE EQUIPMENT (PPE): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Steel Cap Boots |  | Gloves |  | Hard Hat & Glasses |  | Hearing Protection |  |  |  | Hi-Vis |  |  |  |  |  | Dust Mask |  |  |  |  |  |  |  |  | Harness |  | Isolation |  |  |  |  | Hand Washing Hygiene |  |  | Double Eye Protection |  |  |
| RELEVANT LEGISLATION / CODE OF PRACTICE / STANDARDS: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Mining & Quarrying Safety and Health Act |  |  |  |  |  |  | Work Health and Safety Regulation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | COP Managing Risk in the Workplace |  |  |  |  |  |  |  |  | COP Managing the Risk of Falls |  |  |  |  |  |
| Mining & Quarrying Safety & Health Reg |  |  |  |  |  |  | Environmental Protection Act |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | COP Managing Risks of Plant |  |  |  |  |  |  |  |  | COP Risks of Hazardous Chemicals |  |  |  |  |  |
| Work Health and Safety Act |  |  |  |  |  |  | Environment Protection Regulation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | COP Construction Work |  |  |  |  |  |  |  |  | COP Hazardous Manual Tasks |  |  |  |  |  |
| HAZARDOUS CHEMICALS / DANGEROUS GOODS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Chemical(s) to be Used: |  |  |  |  |  |  |  |  |  |  |  | Risk Assessed: |  |  |  |  |  |  |  |  | SDS Available: |  |  |  |  | If “No”, Explain Why: |  |  |  |  |  |  |  | Hazardous/Dangerous: |  |  |  |
| Sodium cyanide (NaCN) / cyanide-bearing slurry and solution |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | No |  |  |  |  |  | Yes |  |  | No |  |  |  |  |  |  |  |  |  | Yes |  | No |  |
| Silver nitrate (AgNO3) titrant |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | No |  |  |  |  |  | Yes |  |  | No |  |  |  |  |  |  |  |  |  | Yes |  | No |  |
|  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | No |  |  |  |  |  | Yes |  |  | No |  |  |  |  |  |  |  |  |  | Yes |  | No |  |
| HIGH RISK WORK: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Excavation / Trenching |  |  |  | Falling from Height |  |  |  |  |  |  | Rigging / Lifting |  |  |  |  |  |  |  |  |  |  | Powered Mobile Plant |  |  |  |  |  |  |  | Scaffolding |  |  |  |  |  |  |  |
| Pressurised Equipment |  |  |  | Asbestos |  |  |  |  |  |  | Demolition |  |  |  |  |  |  |  |  |  |  | Structural Alteration |  |  |  |  |  |  |  | Flammable Atmospheres |  |  |  |  |  |  |  |
| Explosives |  |  |  | Pre-Cast Concrete |  |  |  |  |  |  | Electrical |  |  |  |  |  |  |  |  |  |  | Confined Space |  |  |  |  |  |  |  | Chemical/Fuel/Refridge line |  |  |  |  |  |  |  |
| Extremes of Temperature |  |  |  | Traffic Corridors |  |  |  |  |  |  | Risk of Drowning |  |  |  |  |  |  |  |  |  |  | Driving Work |  |  |  |  |  |  |  | Telecommunication Tower |  |  |  |  |  |  |  |
| PERMITS REQUIRED |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type: |  |  |  |  | Permit Required: |  |  |  | Permit No: |  |  |  |  |  |  |  |  |  | Type: |  |  |  |  |  |  |  |  |  | Permit Required: |  |  |  |  | Permit No: |  |  |  |
| Excavation Permit |  |  |  |  | Yes |  |  | No |  |  |  |  |  |  |  |  |  |  | Working at Heights Permit |  |  |  |  |  |  |  |  |  | Yes |  |  | No |  |  |  |  |  |
| Confined Space Permit |  |  |  |  | Yes |  |  | No |  |  |  |  |  |  |  |  |  |  | Hot Work Permit |  |  |  |  |  |  |  |  |  | Yes |  |  | No |  |  |  |  |  |
| MOBILE PLANT REQUIRED FOR TASK |  |  |  |  |  |  |  |  |  |  |  |  |  | TRAINING REQUIRED FOR TASK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Plant: |  |  |  |  | License: |  |  |  |  |  |  |  |  | Qualification: |  |  |  |  |  |  |  |  |  |  |  |  | Yes: |  | No: |  | Comments: |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | Construction Industry Induction Card |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | Heritage Minerals Site Induction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | Other: |  |  |  |  |  | Cyanide awareness and gas detection competency where the area requires it |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | Other: |  |  |  |  |  | Working at height competency where the task requires it |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | Other: |  |  |  |  |  | Competency in the determinations and calculations carried out in this task |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HERITAGE MINERALS STANDARD OPERATING PROCEDURES: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work Health and Safety SOPs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Environmental SOPs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-001-R02-Legal Obligations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-ENV-SOP-001-R02-Hazardous Substances |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-002-R02-Risk Management |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-ENV-SOP-002-R02-Soil Management |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-003-R02-Consultation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-ENV-SOP-003-R02-Fire Management |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-004-R02-Incident Reporting |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-ENV-SOP-004-R02-Cultural Heritage |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-005-R02-Emergency Response |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-ENV-SOP-005-R02-Dust & Air Management |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-006-R02-Audit & Inspection |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-ENV-SOP-006-R02-Waste Management |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-007-R02-Drugs & Alcohol |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-ENV-SOP-007-R02-Water Management |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-008-R02-Permit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-ENV-SOP-008-R02-Noise & Vibration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-009-R02-Induction & Training |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-ENV-SOP-009-R02-Flora Management |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-010-R02-Working Over Water |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-ENV-SOP-010-R02-Fauna Management |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-011-R02-Working in Isolation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-ENV-SOP-011-R02-Dredging Operations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-012-R02-COVID-19 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Quality SOPs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-013-R02-HAZOP |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-QAL-SOP-001-R02-Management Review |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-014-R02-Change Management |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-QAL-SOP-002-R02-Objectives & Targets |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-015-R02-Electrical |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-QAL-SOP-003-R02-Monitoring & Measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-016-R02-Cranes & Rigging |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-QAL-SOP-004-R02-Document Control |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-017-R02-Working at Heights |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-QAL-SOP-005-R02-Procurement |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-018-R02-Confined Space |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-QAL-SOP-006-R02-Record Management |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-019-R02-Isolation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-QAL-SOP-007-R02-Pre-Planning |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-020-R02-Asbestos |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-QAL-SOP-008-R02-Custom Focus |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-021-R02-Fatigue |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-QAL-SOP-009-R02-Design Management |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-022-R02-Plant & Equipment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-QAL-SOP-010-R02-non-conformance |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-023-R02-Excavation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-QAL-SOP-011-R02-Tendering |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-024-R02-Light Vehicle |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-QAL-SOP-012-R02-Supplier Engagement |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS-SOP-025-R02-Security |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-QAL-SOP-013-R02-HR-Human Resources |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS -SOP-026-R02-Bullying & Harassment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-QAL-SOP-014-R02-HR-Disciplinary |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS -SOP-027-R02-Disciplinary |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-QAL-SOP-015-R02-HR-Bullying & Harassment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HM-MMM-WHS -SOP-028-R02-Issue Resolution |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-QAL-SOP-016-R02-HR-Issue Resolution |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-QAL-SOP-017-R02-Stakeholder Consultation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HM-MMM-QAL-SOP-018-R02-Crisis Management |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | HERITAGE MINERALS 5X5 RISK MATRIX |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RISK RATING: | LOW (1-4) Broadly Acceptable-Manage by Routine SOPs |  |  |  | MODERATE (4-9) Tolerable-with Identified Controls Fully Implemented |  |  | HIGH (10-19) Undesirable-Additional Controls Required to Reduce Risk |  | EXTREME (20-25) Intolerable-Do Not Start Activity |  |
| STOP WORK IF UNSAFE |  |  | CONSEQUENCE |  |  |  |  |  |  |  |  |
|  |  |  | 1 = Insignificant |  |  | 2 = Minor | 3 = Moderate |  | 4 = Major |  | 5 = Catastrophic |
|  |  |  | People-No injuries requiring treatment. Environment-Minor localised environmental harm rectified within hours. No protected habitat or species affected. |  |  | People-First aid treatment. Environment-Minor transient environmental harm that requires days for recovery. No protected habitat or species affected. | People-Medical treatment required, Restricted Work Injury. Environment-Significant environmental harm that requires weeks for recovery. Environmental incident involving protected species/habitat. |  | People-Injuries resulting in; lost time, permanent disability, amputation, or surgery. Environment-Very serious long term environmental harm or contamination, years to recover. Protected species habitat damage. |  | People-Single or multiple fatalities. Environment-Severe environmental harm or contamination resulting in permanent environmental damage. Endangered species and habitat destroyed. |
| LIKELIHOOD |  |  |  |  |  |  |  |  |  |  |  |
| 5 = Almost Certain | Is Expected to Occur |  | MODERATE (5) |  |  | HIGH (10) | HIGH (15) |  | EXTREME (20) |  | EXTREME (25) |
| 4 = Likely | Will Probably Occur in Most Circumstances |  | MODERATE (4) |  |  | MODERATE (8) | HIGH (12) |  | HIGH (16) |  | EXTREME (20) |
| 3 = Possible | Should Occur at Some Time |  | LOW (3) |  |  | MODERATE (6) | MODERATE (9) |  | HIGH (12) |  | HIGH (15) |
| 2 = Unlikely | Could Occur at Some Time |  | LOW (2) |  |  | LOW (4) | MODERATE (6) |  | MODERATE (8) |  | HIGH (10) |
| 1 - Rare | May Occur Only Once in 5 Years |  | LOW (1) |  |  | LOW (2) | LOW (3) |  | MODERATE (4) |  | MODERATE (5) |
| L = | LOW |  |  | Work may proceed after being approved by your supervisor and HSEQ Superintendent, record and review regularly as any equipment / people / work process or SOPs change. |  |  |  |  |  |  |  |
| M = | MODERATE |  |  | Work may proceed after being approved by your supervisor and HSEQ Superintendent, but controls must be monitored and reviewed regularly. |  |  |  |  |  |  |  |
| H = | HIGH |  |  | Work may NOT start report to your supervisor, who will report to site management for further mitigation of risk i.e., increase supervision and inspection, use of permit system, SSE and HSEQ Superintendent must approve. |  |  |  |  |  |  |  |
| E = | EXTREME |  |  | WORK MUST NOT START, report to you supervisor, the risk must be re-assessed to an acceptable level. Further controls are to include Increase supervision, inspection, permit system must be used, risk assessment workshop, SSE and HSEQ Superintendent must approve. |  |  |  |  |  |  |  |

| Likelihood (L): | 1 = Rare, 2 = Unlikely, 3 = Possible, 4 = Likely, 5 = Almost Certain |  |  |  |  | Consequence (C): |  | 1 = Insignificant, 2 = Minor, 3 = Moderate, 4 = Major, 5 = Catastrophic |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Risk Rating: | LOW (1-4) Broadly Acceptable-Manage by Routine SOPs |  | MODERATE (4-9) Tolerable-with Identified Controls Fully Implemented |  |  |  | HIGH (10-19) Undesirable-Additional Controls Required to Reduce Risk |  |  |  |  |  | EXTREME (20-25) Intolerable-Do Not Start Activity |  |  |  |  |  |  |
| Task / Step No. (Job Steps) | Potential Hazards (What Can Go Wrong) | Initial Risk (L x C=R) |  |  | Control Measures |  |  |  | Residual Risk (L x C=R) |  |  | Hierarchy of Controls Applied |  |  |  |  |  |  | Responsible Workers |
|  |  | L | C | R |  |  |  |  | L | C | R | Elimination |  | Substitution | Isolation | Engineering | Admin | PPE |  |
| 1. This task cannot be completed. See the folder README for the blocker. | - Hydrogen cyanide gas released at the open sample point | 3 | 5 | H15 | - Personal HCN monitor worn and switched on, in calibration date - Fixed detection confirmed healthy - alarm 5 ppm, high-high 10 ppm - Sample point approached from upwind. pH of the stream confirmed above 10.5 before opening - Second person in sight and clear - Cyanide antidote kit on site and a trained first aider on shift - Withdraw upwind and report on any reading above the site action level |  |  |  | 1 | 5 | M5 |  |  |  |  | x | x | x | Met Lab Technician |
| 2. The document is issued so the register line is not empty, and is marked NOT FOR USE. | - Hydrogen cyanide gas released at the open sample point | 3 | 5 | H15 | - Personal HCN monitor worn and switched on, in calibration date - Fixed detection confirmed healthy - alarm 5 ppm, high-high 10 ppm - Sample point approached from upwind. pH of the stream confirmed above 10.5 before opening - Second person in sight and clear - Cyanide antidote kit on site and a trained first aider on shift - Withdraw upwind and report on any reading above the site action level |  |  |  | 1 | 5 | M5 |  |  |  |  | x | x | x | Met Lab Technician |

| EMERGENCY RESPONSE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Nearest Emergency Assembly Point? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Nearest Fire Extinguishers? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Nearest First Aid Kit? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Method of Communicating an Emergency? |  |  |  |  |  |  | Phone No.: |  |  |  |  |  |  |  |  |  |  | UHF Channell: |  |  |  |  |  |  |  |  |
| ONGOING MANAGEMENT REVIEW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Review No: |  | 01 |  |  | 02 |  |  | 03 |  |  |  | 04 |  | 05 |  |  | 06 |  |  | 07 |  | 08 |  |  |  | 09 |
| Name: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Date: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Initial: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WORKERS CONSULTED ON DEVELOPMENT OF JSEA/SWMS: E.G. WORKERS, SAFETY REPRESENTATIVE, SUBJECT MATTER EXPERT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Name: |  |  |  | Position: |  |  |  |  | Name: |  |  |  |  | Position: |  |  |  |  | Name: |  |  |  | Position: |  |  |  |
| APPROVAL (Can be Approved by SSE or Supervisor or HSEQ Superintendent) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Name: |  |  |  | Position: |  |  |  |  | Date: |  |  |  |  | Name: |  |  |  |  | Position: |  |  |  | Date: |  |  |  |
| CHANGE MANAGEMENT PAGE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Date and Time: |  |  | Step #: |  |  | Description of Change: |  |  |  |  |  |  |  |  | New Control Measures: |  |  |  |  |  |  |  |  | Change Approval: |  |  |
| SIGN ON PAGE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Date: | Full Name: |  |  |  |  |  |  |  |  |  | Signature: |  | Date: |  |  | Full Name: |  |  |  |  |  |  |  |  | Signature: |  |